Operation

Refer to the Operating Manual and Errata sheet for  firmware

commands and operating instructions. PacComm firmware is derived from

TAPR and we have maintained a strong similarity in the command structure.

Use of the DSP-93 TNC should appear very similar to operating a TNC-2.

Baud Rates

The data rates on the three TNC ports are set by the BAUD command.

The terminal defaults to 9,600, 8, N, 1.

Pass Through Function

If the connection was made to the DSP-93 at U21, the TNC's serial port

may be tied to the DSP serial port through use of the PASSTHRU mode. This

will allow loading the DSP code via the TNC's serial port, and allow DSP

output which is not HDLC to be passed unchanged to the terminal.

The PASSTHRU mode is entered by giving the command PASSThru ON.  When ON,

all bits entering the TNC serial port are transferred out the second

serial port (DSP-93 load port) and vice versa. When PASSTHRU is OFF the

second serial port is ignored. Pin 11 of U21 must be grounded to disable

DSP-93 flow control.

PASSTHRU is a fully transparent mode. Any bit pattern may be sent either

direction without causing any effect on the TNC's command state or

operation.

To get out of PASSTHRU mode, use the same scheme as is used for exiting

Transparent mode, e.g. one second of silence, three CTRL-C within one

second, and one additional second of silence.

Other Features

Parallel I/O

Eight parallel I/O lines are available at J4 for user-specific

applications. These lines are controlled via the IN and OUT commands.

Connection to J4 is left to the user. It is recommended that opto-

isolators or other buffering be provided to prevent damage to the

processor I/O lines. If such damage is sustained, the entire processor IC

must be replaced.

Care should be taken in using the IN and OUT commands, especially the OUT

command. If one of the processor internal registers is accidentally

specified it is possible to crash the firmware. This may require a hard

reset and all parameters will be lost.

Analog-to-Digital Converter

The circuit board is wired to accept a PacComm AD-4/Header analog-to-

digital converter board at J2 as part of the AVL project.  There is no

support for the A-D converter in the EPROM at this time.  Contact PacComm

if you have further interest in this.

Notes

Heat-Stroke on the DSP-93.

Some DSP-93 owners have reported their units suffering from heat related

problems. These problems have been analyzed by TAPR and solutions found.

Adding the TNC into the case should not cause a heat or power problem.

PacComm Operating Manual Errata and Updates, 2/26/96

The latest version of the Operating Manual Operating Manual for PacComm

Packet Controllers, 6th Ed., 1st Printing, May, 1994.

The TNC for the DSP-93 version of PacComm packet firmware is labeled

"Release 3.2.9.3  2/11/96."

The BAUD command is somewhat different than shown in the Operating

Manual.

KISS mode is fully implemented. NOTE: There were several occurances of

PTT lockup (transmitter keyed on) at one site during testing. We believe

this was due to RF feedback, because we have not been able to re-create

this problem at other test sites. It is recommended that KISS mode not be

used for unattended operation until you have used it sufficiently to feel

confident of its proper operation in your setup.

The CAL function (tune) does not work correctly.

RAM greater than 32k is not yet supported.

The TNC will not do a digital or analog loopback. Test only by connecting

to or via another packet controller.

New features in Release 3.2.9.1 and later releases not covered in the

manual:

a) The monitor command remains operative when GPS is set to ON, i.e. the

monitor function is on or off depending on the setting of MON regardless

of whether GPS is on or off regardless of the setting of GPS.

b)The MYROSE parameter holds the callsign of the local ROSE switch.  Then

a connect command to the ROSE network may be made in the form:

C Call @ Destination Switch<R>

The connect request will be properly reformatted and sent to the local

switch.

The following new commands are in this pre-release of 3.2.9.3 code:

MY1alias, MY2alias, MY3alias are additional callsign alias fields.

MYAlias remains unchanged.

PRefix specifies a character which will be prefixed to the beginning of

each 'monitored' frame. It is used by some type of terminal programs to

segregate monitored frames from connected data. The default value is $00

which adds no character to each frame.

ELOc (Echo Location) is a PicoPacket command which was included by

mistake. It should be left OFF.

EPROM Changes Planned

The PASSTHRU function loses characters from the DSP-93. This will be

corrected in a future release.

Suggestions are welcomed.

Support

You may obtain support for this product via any of the following methods:

Email to the TAPR DSP-93 SIG. PacComm monitors this SIG continuously.

Contact TAPR for instructions on subscribing to this SIG.

Future EPROM releases will be available in the TAPR DSP-93 firmware

library or via direct from PacComm.

Email direct: dsptnc@paccomm.com.

Compuserve: 76576,2003.

PacComm BBS: +(813) 874-3078.

PacComm facsimile: +(813) 872-8696.

PacComm voice technical support: +(813) 874-2980.

PacComm Packet Radio Systems, Inc.<R>

4413 N. Hesperides St., Tampa, FL 33614-7618 USA

More information about the DSP-93 project may be obtained by ftp from

tapr.org. Use anonymous login and use your IP address as the password.