TNC set to 9600 baud!

=================================

TPRSP1.OBJ - Radio Port 1, gain 4

TPRSP2.OBJ - Radio Port 2, gain 4

=================================

To assemble, use the command "asm fsk.asm -dRn -dGn"

where Rn is R1 or R2 for radio ports 1 or 2 and

where Gn is one of G0, G1, G2, G3, G4, G5, or G6 to set the gain

9600 bps full-duplex FSK modem, Texas Packet Radio Society (TPRS) compatible.

This version is basically the same code as FSK.OBJ, but with scrambler and

descrambler mods for use with the TPRS network.

Because the TPRS protocol is polarity dependent, the code may depend on

the polarity of your transceivers detector and modulator. You may have to

experiment with the following lines in the source code.

To change the polarity of the receiver demodulation, change the values

loaded at labels SOHI and SOLO. Choose one of the following at each label.

SOHI      LACK      01h       ;  else load 1

SOHI      ZAC                 ;  load 0

SOLO      ZAC                 ;  load 0

SOLO      LACK      01h       ;  else load 1

To change the polarity of the transmitter modulation, change the values

loaded at labels SOHI and SOLO. Choose one of the following at each label.

T_HI      LACK      01h       ;  load 1

T_HI      ZAC                 ;  load 0

T_LO      ZAC                 ;  load 0

T_LO     LACK      01h       ;  load 1

See the .ASM file for more information

License and Disclaimer

A no cost, non-exclusive license to use the enclosed software is granted

to all users of the TAPR/AMSAT DSP-93 DSP computer, with the restriction

that it be used only for non-commercial amateur radio applications. ALL

other uses are prohibited under this license. Copyright 1991, 1992, 1993

and 1994, Frank H. Perkins, Jr., WB5IPM. All rights reserved.

This software is EXPERIMENTAL. It is offered AS IS, without Warranty or

Guarantee of any kind. NO representation is made that this software is

suitable for any purpose, or that its operation is reliable. Use this software

strictly at your own risk. The author, AMSAT and TAPR accept no liability

for any loss or injury arising directly or indirectly from its use.

Never use this software under circumstances where its failure to operate

or its failure to operate as expected, or the failure of the attendant

hardware could create a dangerous situation, cause loss of data, or could

cause damage to your amateur radio equipment, computer equipment, or

any other equipment or property.

Please contact Greg Jones at TAPR or the author for information on

commercial licenses of this software.

Frank H. Perkins, Jr.

#######