From:   Paul Newland, ad7i

To:     METCON Users

Date:   930117

Subj:   METCON-1 Firmware Errata for Version 930117-1.07

OVERVIEW

A new version of METCON-1 firmware has been released.  METCON-1

firmware version 1.07 cleans up some discrepencies in 1.06.  The

Fast Load Up commands now send more bytes and instead of sending the

map display on an auto-timeout a Fast Load Up 0 can be sent instead

(see AFLU0 flag bit).

REPEATED CAUTION

To make the changes above some "crunching" of the code was needed to

create more space.  We were able to get the changes into the 4 K

code limit without dropping features.  However, the next time we do

this something has got to go.  Since the BIT-SET (=AS093), BIT-CLEAR

(=AC093), and BIT-DISPLAY (=AD093) commands are redundant to the

BIT-WRITE and BIT-READ commands, we will no longer support the

BIT-SET, BIT-READ and BIT-DISPLAY commands.  The SET/READ/DISPLAY

commands are functional in version 1.06 but we will no longer

support them in any documentaion.  Additionally, the next time we

need more code space to correct bugs or add needed features, the

SET/CLEAR/DISPLAY commands will be removed from the code.  The same

functionality is provided through the BIT WRITE/READ commands; these

will NOT go away.  Going forward, please use the BIT WRITE/READ

commands instead of the BIT SET/CLEAR/DISPLAY commands so that it

won't come as a shock some time in the future if the

SET/CLEAR/DISPLAY commands go away.

OPEN USER MEMORY FLAG CHANGES

A new flag as been added, it's called AFLU0

AFLU0 (Badr:003)

When this bit is one the automatic display will be replaced with a

Fast Load Up 0 output.  Note this flag will not be effective unless

either AD1 or AD15 are set to one.

The default value of AFLU0 is zero.

METCON REVISION HISTORY

910326-1.00             Initial Release

910731-1.01             First Revision

Fix binary to Decimal Coversion

Add Serious Authentication System

Iss 0

METCON-1 1.07 Errata            -2-                           930117

Add Auto Disconnect if Not Logged In w/in 60 sec

910907-1.02             Second Revision

Make Byte Read/Write give data in Decimal, not just Hex

and Binary

Add Accumulating Pulse Counters

Correct Authentication Problem When Not SYSADMIN

Move ECHO and NLF to Restricted Memory

Change ECHO (active high) to NECHO (disable when high)

911125-1.03            Third Revision

In file IO.ASM added code to UDP0 to clear change bits

for all inputs on port 0 that are configured for

accumulating pulse counters

920405-1.04            Fourth Revision

Change sequence of startup if EEPROM failure detected

Provide individual CHANGE bits for each input

Move NII and NIO to restricted memory

Only accumulate pulse values on those ports that are

configured as pulse counters.  In version 1.02 and

1.03 an input configured for pulse counter worked fine.

However, inputs configured as frequency counters

contained a value equal to the frequency PLUS the

number of pulses counted (sampled at a 20 samples

per second rate).  A work-around for 1.02 and 1.03

is to set the input ports used for frequency

counters to 1 (i.e, cmd =AS4x where x is 0-7, do

for each input used as frequency or pulse ctr).

921031-1.05            Fifth Revision

Add station ID to Fast Upload Response

Provide response to =AED command (-A- OK)

Provide 20 Hz signal on T1 output for test purposes

Allow user to select value of UART D7 output

Move major system flags one byte lower in memory map

921031-1.06             Sixth Revision

Change Date (no other code chages since 1.05)

Change release notes regarding non-support of BIT SET/CLEAR/DISPLAY

930117-1.07             Seventh Revision

Correct Number of Bytes sent for FLU0 and FLU1

Added new flag bit, AFLU0, (Badr 003) to select Fast Load

Up block 0 as an alternative to the memory map display

---ooOoo---

Iss 0