SHOWTIME.EXE

Version 2.85

Tom Clark, W3IWI

clark@tomcat.gsfc.nasa.gov

- or -

w3iwi@amsat.org

Dec. 1, 1995

I have written the program SHOWTIME.EXE to provide a number of display

functions for the MOTOROLA PVT-6 "ONCORE" receivers that are being used for

high accuracy timing by both the Space Geodesy and Amateur Radio communi-

ties in a project that I have dubbed the "Totally Accurate Clock" ("TAC",

which I immodestly note happens to be my initials!).

SHOWTIME is a stand-alone program designed to run on an IBM-PC clone and is

intended to provide a number of useful display and control functions. When

you run it, you will see that the screen is divided into 3 general areas:

- The top half (the TIME screen) displays the current time and date

information. The most striking feature is the HH:MM:SS field which

has BIG characters that update once per second. You have the option

of displaying either UTC or local time. The bottom part of the TIME

screen is called the CALENDAR area where the current UTC date, day-of-

year, day-of-week, GPS week, modified Julian Date (MJD) and both the

Greenwich and Local mean sidereal times (GMST and LMST) are displayed.

Also displayed (optionally) is information on the 1 pulse-per-second

(1PPS) output timing (with a resolution of 1 nsec) and an estimate

of the current accuracy of the 1 PPS signal.

- The bottom half of the screen is divided into two windows. The lower right-

hand quadrant (the SAT screen) displays information on the satellites that

are currently in view. For all satellites in view (defined by the current

elevation mask, up to a maximum of 12), the current azimuth and elevation

are shown, along with an arrow showing whether the satellite is rising or

setting. For the satellites that are locked, the SNR is also displayed.

The SNR (in approximate dB) is given both digitally and in a simple bar-

graph "S-meter" format. The lower right-hand corner of the SAT window also

shows a tally of the satellites in view, locked and in use.

- The lower left-hand quadrant half does double duty. In normal operation the

display shows information on receiver status (tracking modes, xDOP, etc),

and either position or timing data (the POS screen). In this screen you will

see the fixed reference position, the long-term average position and the last

position reported by the GPS receiver (at a 1/second rate).

- The lower left-hand quadrant also functions as a help screen and for

parameter input for menu-driven commands. The basic menu is activated

by hitting the SPACE key, and the POS screen is restored with the

<esc> key. Hopefully the menu keys are fairly self-explanatory!

------------------------------------------------------------------------------

Getting started:

Two files (in addition to SHOWTIME.EXE) are required to make SHOWTIME work:

1. A *.GPS control file containing at least an approximate Lat/Lon/Height

position (which should be accurate to 1 degree initially). The details

of this file are described below. SHOWTIME can re-write the control

file as you change parameters. The default file name is SHOWTIME.GPS

2. A read-only file named SHOWTIME.TAC which contains a legal disclaimer

that I felt needed to be included.

These two files MUST be in the MSDOS subdirectory that you start SHOWTIME from.

Don't try using a PATH or DOS ENVIRONMENT statement. I put the .EXE files in

the \TAC\BIN directory and then have the files SHOWTIME.GPS, SHOWTIME.TAC and

SHOWTIME.BAT file in my \TAC directory. The SHOWTIME distribution software disks

set up this directory structure for you by running the SETUP file like

A:\>  SETUP C:

If you have already installed SHOWTIME with the SETUP.BAT file, then use the

UPDATE.BAT file by typing

A:\>  UPDATE C:

The latest version of SHOWTIME can be obtained by anonymous ftp from

ftp://aleph.gsfc.nasa.gov/GPS/totally.accurate.clock

where you will find files named like SHOWTM28.ZIP, SETUP.BAT and UPDATE.BAT.

If you need a complete distribution disk, you can fetch a complete image of

my distribution floppies from the aleph host in the ~/distribution directory

where you will find files named like tac.951024.distrib.zip. Use a binary

get to bring this file to your PC as a temporary file (like temp.zip):

ftp:  binary

ftp:  get tac.951024.distrib.zip   temp.zip

Then use PKUNZIP to make an installation floppy, usind the -d option to

create the directory structure on the floppy:

pkunzip -d temp a:

-----------------------------------------------------------------------------

SHOWTIME assumes that you have a MOTOROLA PVT-6 "ONCORE" receiver connected

to either the COM1 or COM2 port. You specify COM1/2 by either of 2 ways:

a. Include it in the *.GPS set-up file described below

b. Do nothing and you will get COM1 as a default

The PVT-6 "ONCORE" receiver MUST be equipped with the timing option, which

MOTOROLA denotes as OPTION A (and which costs about $100 extra). The 1 PPS

signal needs to be supplied to the PC on the COMx DCD handshaking line.

As a part of our "TAC" project, I designed a piggyback board to mount on top

of the receiver which supplies this function (along with a number of other

"neat" things which I won't discuss here).

Nominally this signal should be at RS232 levels, but it will work on most

PCs by connecting the receiver's CMOS-level (0/+5v) 1 PPS signal directly to

the COM1/2 RS232 connector on most PC's; if your PC works OK with 0/+5 volt

signals, then the following minimal configuration works for an IBM standard

9-pin RS232 connection. The battery (a 9V transistor radio batter works fine)

is optional but highly recommended.

CAVEAT EMPTOR #1: Use either the +5V  OR  the +12V power input BUT NOT BOTH!!

Also note that I have followed MOTOROLA's pin numbering scheme on the PVT-6

connector; this is different from the normal numbering sequence which has ODD

pins on one side of the connector and EVEN pins on the other. MOTOROLA denotes

their pin #7 as "1PPS Return" -- however the Power Ground, 1 PPS Return and

Data Return (pins #3, #7 and #10) are connected together on the receiver board.

CAVEAT EMPTOR #2: DO NOT use these connections for the smaller Motorola "ONCORE

VP" receiver board. While the pin numbers are similar to the PVT-6 "ONCORE

BASIC", Motorola decided to use a different (and more conventional) way to

number the pins (odd numbers on one side, even on the other) and you WILL burn

up the receiver if you use the connections shown below !!!!

top  PVT6 bottom        PC's RS232

5  10               (9-pin)

+12 VDC------o  o---------------o 5 = data gnd

o  o------<--------o 3 = TXD to GPS

Power Gnd----o  o------>--------o 2 = RXD from GPS

+5 VDC-------o  o

+ Battery----o  o------>--------o 1 = DCD input

1  6

----------------------------------------------------------------

To run SHOWTIME what to do, the command is

SHOWTIME xxx.GPS    or    SHOWTIME ?

[?] will display a brief sign-on message. If you include a [?], then

SHOWTIME halts after displaying the message.

[xxx.GPS] denotes the name of a control file (default = SHOWTIME.GPS)

which contains all normal operating parameters for the receiver. At a

minimum, your startup file contain at least the three lines:

LON:  �dd.ddd      = your approximate longitude (-West, +East)

LAT:  �dd.ddd      = your approximate latitude

HGT:  �mmm         = your approximate height

which will be used to "seed" the receiver and get things started.

Other parameters have reasonable defaults built-in to SHOWTIME. After

SHOWTIME is running you can bring up the [F]iles menu and [U]pdate a

complete list of operating parameters in whatever file you choose. If

you want to be able to restart a particular operating configuration,

you can choose from amongst any of the different files you have saved.

The xxx.GPS file contains a number of commands. If you enter the list manu-

ally, they can be in any order. When you tell SHOWTIME to write a file (with

the [F] command), they are in a fixed order and the resulting file looks

something like this -- hopefully most of the entries are self explanatory!

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

#### � SHOWTIME v2.85 �  File

####  1 Dec. 1995 = #310 + 02:59:54.0

#### Posns:  Ref             Avg            Delta    RMS     #Sec

LAT:     +39.1883333333   +39.1883756070    +4.697   6.89    9065

LON:     -76.9350833333   -76.9351177120    -2.961   3.49    9065

HGT:         +100.000         +110.546     +10.546  17.06    9059  Geoid +34.0

REFID    Testing SHOWTIME v2.80

#### Control                 ----------+----------

COM2                                   |COM1/2

TZ:      UTC  0                        |Time Zones: PC

LTZ:     EST 5                         |  Local

TimeSwap EST                           |  Initial

AUTOSET  0                             |AutoSet rate

XTIME    .85                           |Time Seq Slice

LOCKHYST 2                             |Alarm Hyst,s

PCRATE   1                             |PC Update,1-3s

#### Receiver                ----------+----------

NDim     3                             |0/2/3-D

ELEV     1                             |Min El

AIV                                    |All-in-View

PDOP                                   |xDOP Selection

HIGHest in Sky                         |not PDOP

APtype   4                             |=Fixd

IGNORE  +++++++++++++++++++�+++++++++++|Accept=+

; PRN=31^       21^       11^        1^| PRN31=>1

GEOID    0                             |Corrections:

IONO     1                             |   Geoid:N Ion:Y

#### Avg Window Length       ----------+----------

POS      43200                         |Pos

RMS      43200                         |RMS

SigmaT   43200                         |SigmaT

DOPWeight                              |DOP weighted positions

Tsigma: 100.5   58.6   64.2       9058 |max/min/avg Tsigma,#Sec

Locked:  #309 + 23:50:36.0          1  |Since,#Unlock

#### Timing                  ----------+----------

Epoch    0                             |Epoch Offset

Cable    92.4                          |Cables,ns

Inst     8                             |Rcvr Delay,ns

Early    0                             |Early offset,ns

#### Ticks                   ----------+----------

Stone    1000                          |Sec tick,Hz

Mtone    1200                          |Min tone,Hz

SLong    0                             |Min tone@sec

#### Colors in order:        ----------+----------

##  BkGnd,BigUTC,Calndr,SatInfo,Epoch,Faint,Frame

##  Msec Offset,Posn,DOP/Stat,Help,Red Warning

Colors: 1  14  10  10  12  9  8  7  6  13  14  12

#### END OF FILE

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

If you have an incomplete *.GPS file, then SHOWTIME will supply suitable

default values. When you re-write the file to disk (from the [F] menu), the

new file will will contain all the parameters. The entries in the list can

be in any order but will be re-written in fixed format. Lines starting with

#'; or a blank character are ignored. The numeric data can be in any column

providing a couple of blanks are between the command and the data (the file

written by the machine has most data starting in column 10). Except for the

longer lines (like LAT:/LON:/HGT:/Colors:), columns to the right of 40 are

not parsed and are intended for text comments. Some commands are a single

key-word with no numeric parameters. As an example, AIV/BEST4 are alter-

natives for the receiver operating mode. Some parameters need additional

explanation:

-  [TZ: xxx #] and [LTZ: xxx #]:  SHOWTIME includes several clock display

options, and so it keeps three different times internally. The TZ

parameters are used to set your PC's internal clock to high-accuracy

GPS time. The numeric part of the TZ parameter specifies the offset of

you PC's clock from UTC in hours earlier than UTC. The format is

therefore like  TZ: UTC 0  or  TZ: EST 5  or  TZ: MET 23 ( or MET -1),

with a 3- or 4- character time-zone designation followed by the UTC

offset in hours. (Personally, I keep my PC running in UTC, as you can

see from the example above.)

-  The LTZ parameter follows the same conventions for an alternate Local

time display. At any time you can hit the [!] key and swap the main

time display window between UTC and Local time. When you select Local

time, then the Calendar window will display the time that your PC is

keeping. The Calendar portion of the screen shows only the UTC date,

day-of-year, day of the week, etc.

-  The TimeSwap parameter lets you decide on whether the intialization

routine starts in UTC time or your local time as defined by the LTZ

parameter. When you [F]ile [U]pdate the current position of the [!]

switch is saved as TimeSwap. If the TimeSwap line is missing from the

file, or if the line reads  "TimeSwap UTC", then the display clock

initializes in UTC time. If the line reads "TimeSwap  anythingelse",

then the default is given by the LTZ parameter.

-  [EXIT or EXIT BINARY]: When you exit SHOWTIME with the

[X] command, the EXIT switch determines if the receiver is left running

in 4800 baud ASCII NMEA mode or in 9600 baud Motorola Binary mode. (The

Motorola controller software will run OK whichever position is used,

but it takes longer to initialize if the receiver was left running in

4800 baud NMEA mode). The next time SHOWTIME is run, it doesn't care

which mode the receiver was left in and timing functions set up by

SHOWTIME continue to function after you exit. If the EXIT line is

omitted from the file or just EXIT appears, then the default is 4800

baud NMEA format the message rates used in SHOWTIME. Prior to Version

2.50, and additional option "EXIT NMEA" allowed you to specify the

rates of NMEA messages upon exit. This was dropped in Version 2.50.

-  [STONE, MTONE and SLONG]: To make it easier to manually set the seconds

on another clock (like the Mark-3 VLBI formatter or your wrist-watch)

SHOWTIME can generate WWV-like audible time ticks, which are toggled

with the [N]oise key. STONE (default 1000 Hz) is the frequency in Hz

for the one-second ticks. You may need to change it if your PC has a

really bad speaker! The [N]oise ticks can be turned on so that a longer

tick with a different MTONE (default 1200 Hz) pitch on the SLONG second

(default 0) of the minute. SLONG can be changed in the [T]iming menu,

but you must set STONE and MTONE in the xxx.GPS configuration file.

-  Most of my testing was done on a 486/66 machine. If you have a much

slower machine, you may want to try tuning the XTIME parameter --

XTIME is the fractional second after the last tick that the software

quits looking for RS232 data and updates the screen (see next

section). You can also insert a command PCRATE = 1,2 or 3 into the

control file. If PCRATE > 1, then the NMEA GGA/GSA/GSV messages

are sent by the receiver to once every 2 or 3 seconds. Because of

vagaries of the PVT-6 ONCORE receiver, I have not been able to

develop a foolproof algorithm that guarantees that this will work.

Most of the time, if PCRATE=2 then the shorter GGA/GSA messages are

in one second and the longer GSV messages are in another second,

but the assignment is not perfect. Similarly, if PCRATE=3 the 3

messages are supposed to be in different seconds. To see how the

roll of the dice worked, hit the [&] key to see the time slicing

in action. Two numbers are displayed -- the first is the number of

10 msec "slices" that have elapsed, the second is the number of times

passing thru the inner loop waiting for an NMEA "sentence" to arrive.

Type [%] to re-roll the dice if not OK. PCRATE and XTIME can be

set from either the xxx.GPS file or from the [G]PS screen. PCRATE

is displayed and can be set from the [A]veraging Parameters screen.

-  The parameter line like "IGNORE ++++++�+++" (and the comment line which

follows it in the example above) are new in SHOWTIME Ver 2.50. This line

(and the manual entry with [G]PS Ignore/Use in the [G]PS Mode screen)

shows which satellites are to be used/ignored. The "+++�++++" field

has 31 characters in it, corresponding to GPS PRNs 31 thru 01 (left-

to-right)  When SHOWTIME reads this line, it takes "+" to enable a

satellite as good. Any other character (like "-" or "�" or a space)

will disable the satellite. For the example shown above, PRNs 12, 22

and 27 have been turned off.

{Note: The IGNORE switch does not work for PRN32 due to the fact

that a 32-bit un-signed number (1 bit/satellite) needs to be sent to

the Motorola receiver, but Microsoft only allows for 32-bit SIGNED

long integers and I didn't want to take the effort to kludge a full

32-bit solution! Perhaps this will happen when SHOWTIME is implemented

in a REAL computer language!}.

-  [DCDmask #] -- Note -- this function deleted in V2.60. SHOWTIME now

REQUIRES the RS232 1PPS to be on the DCD pin (pin #1 on a 9-pin RS232

connector) and DCDmask is hardwired to &80H in the SHOWTIME software.

--------------------------------------------------------------------

WHAT DOES SHOWTIME DO?

The software initially collects together all the parameters it needs and

then starts running. The first thing is to make certain that it sees the

1PPS signal from the GPS receiver. It takes about 5 seconds to look for a

pattern on the appropriate I/O pin (COM1/2 base address = &H3F8/&H2F8 +6 is

the status port I/O address, and the port data is ANDed with DCDmask). Then

SHOWTIME initializes communications with the GPS receiver. This involves

sending receiver parameters at 9600 baud in Motorola binary format, and then

listening for responses as 4800 baud ASCII NMEA0183 messages.

For various reasons (too mentionable to be numerous) I chose to have SHOWTIME

get data from the GPS receiver using NMEA0183 ASCII strings (specifically, it

uses the $GPZDA, $GPGGA, $GPGSA and $GPGSV NMEA "sentences") at 4800 baud.

However detailed control of the receiver's parameters and switches must be

done at 9600 baud with MOTOROLA's proprietary binary protocol. Both at startup

and whenever you want to make a fundamental change in the receiver config-

uration, it is necessary to toggle between the two speeds -- a nuisance, but

not insurmountable! (Maybe someday this code will be re-written to only use

the 9600 baud binary protocol, but for now we just accept the annoyance!)

A lot of work was done to accommodate the detailed timing of the PVT6 re-

ceiver outputs. The receiver's 1 PPS "tick" occurs on the UTC second (delayed

by a user-defined offset to be discussed later) and lasts for 200 msec; the

accuracy of the leading edge of the 1 PPS signal is what the "TAC" project is

all about! About 50-100 msec after the start of the 1PPS tick, the receiver

starts sending data (time, date, status, etc.) on its RS232 port. The time it

sends lags the pulse and is akin to an announcement on WWV "At the tone, the

time WAS xx:xx". The RS232 data continues for a large fraction of the second,

with the sequencing order of the messages dependent on the details of how the

receiver was initially set up (and hence the order is unpredictable).

The different NMEA messages sent each second are each terminated by a <cr><lf>

sequence and range in length from about 40 and 80 bytes. In SHOWTIME, we use 4

different NMEA messages, each sent once per second:

$GPZDA:  HH,MM,SS,YEAR,MONTH and DAY data

$GPGGA:  position data and status information

$GPGSA:  PDOP, HDOP, VDOP and other status data

$GPGSV:  AZ, EL and SNR data for each satellite in view

Each $GPGSV message gives data on only 4 satellites, so there may be 1,2 or

3 such messages each second, depending on the number of satellites in view

(i.e. above the Elevation mask limit), up to a maximum of 12.

Therefore each second we have at least 4 and sometimes as many as 7 NMEA

messages to parse. The messages average 50-60 bytes long (about 500-550 bits,

since each async byte has a start and stop bit) and hence take 100-120 msec

each. Therefore when the maximum of 7 messages are occurring, nearly 800 msec

are consumed in data transfer. The software doesn't care in what order the

ZDA/GGA/GSA/GSV messages arrive; the important HH:MM:SS data is taken from

the first valid ZDA or GGA message to be sent each second.

The inner loop of SHOWTIME is a big "chain" which is paced by the 1PPS GPS

signal and the PC's 55 msec internal clock. About 10 msec after the "tick",

the software starts looking for NMEA data terminated by the  <lf>  character --

each NMEA sentence ends with a "* checksum <cr> <lf>" sequence. As a line is

received, it is parsed for content and some brief computations are done if the

data is valid; as the software has evolved, a number of validity tests have

been added. The software then waits for another NMEA message terminated by <lf>.

At XTIME = 850 msec (default) since the last 1 PPS tick (measured using the

PC's internal 55 msec clock) we stop looking for RS232 input. The time read

from the GPS receiver in this second is incremented to the upcoming second,

a number of "calendar" and status calculations are done, and the SAT and POS

screens are updated.

{ Note -- the best time for the mode change has been experimentally deter-

mined to be about 0.85 seconds after the 1 PPS tick. Your PC may

be different. The parameter is called XTIME in the xxx.GPS setup file.

It can be changed in SHOWTIME with the [^] (i.e. the up-arrow or carat)

command in the [G]PS menu. You can see a "stopwatch" that shows the timing

of the incoming data by using the main-menu key [&] to show (in 10 msec

units, on the top line of the screen) when RS232 data is being received.

The [&] "stopwatch" also displays the number of passages thru the inner

timing loop waiting for NMEA "sentences" to arrive from the receiver. }

After XTIME, the PC then waits for the leading edge of the 1 PPS "tick" on the

DCD pin of the RS232 connector to do time-critical tasks. These include posting

the new time and calendar data to the screen, making a "WWV-like" tick sound

(if requested), accurately setting the PC's internal clock, etc.

After doing these "on the second" tasks, SHOWTIME then looks to see if a

keyboard entry is present. If a menu key has been struck, the appropriate

action is taken. Finally, SHOWTIME loops back to look for the next second's

NMEA data arriving from the GPS receiver. The input buffer on the COMx port

is big enough so that if some data (especially GSV information) arrived

after XTIME, it is processed at the start of the new second.

The various single-key menu requests set logical switches so that appropriate

tasks are enabled in the once/second timing loop. Because the keyboard is only

polled once/second, the response will seem sluggish. However you will quickly

learn that you can hit several keys in succession and not lose manual input

data. The most important single keys are the  SPACE BAR  (which activates the

main Help menu) and   <esc>   (which deactivates the Help menu).

Some menu requests are for operations that will change the receiver config-

uration and/or foul up the synchronization of the GPS => PC communications.

These include changes to basic receiver parameters (timing, positions and

operating modes) and disk file operations. For all these requests, you are

asked to respond with a [Y]es in order to break the communications link.

On completion of the "off-line" operations, the GPS => PC communications

are re-established and the complete receiver parameter set is loaded.

The high-accuracy timing pulse from the receiver continues with no change

during any of the parameter change operations. Only when you see the screen

clear and "Init:" appears will the changes actually effect the receiver.

At any time you can stop SHOWTIME (or unplug the PC from the "Totally

Accurate Clock") and the timing pulse will be unaffected. The computer is

NOT required for normal operation -- its only function is to control the

receiver and to display time and status information.

For various reasons, the SHOWTIME code is written in MicroSoft QuickBASIC and

is therefore subject to some annoying limitations! The distribution version of

SHOWTIME is compiled as a stand-alone  .EXE  file using the QuickBASIC (v4.5)

compiler. As I was writing the code, I kept adding features until it was at

the limit that QuickBasic's compiler could handle (�65.5k of executable code,

�65.5k of parameter storage, since I could never figure how to get QuickBasic

to handle large code models!  Clearly the next version should be written in

a real programming language!). The distribution version is smaller (about 83 k)

than the "real" 65.5k+65.5k since the distribution SHOWTIME.EXE file is squeezed

with the PKLITE utility.

Initial SHOWTIME software development/testing was done on my personal 486 machine

(486 DX2/66) under MSDOS 6.2 and PCDOS 7.0. It should not need a 486's CPU

power, but I have done little testing on slower machines. I tried the code on

an antique Toshiba T1000 (8088) laptop and it did not work at all. It runs OK on

an old Toshiba 3100 (80286) laptop with the PCRATE=3 switch enabled. The code

runs as a background task under DESQVIEW -- I have even run two copies on

different receivers at the same time. It has been reported that it runs OK in

IBM OS/2 WARP. I have been able to make it run marginally as a DOS application

in WINDOZE 3.1-- you can play SOLITARE and see SHOWTIME at the same time!

Recent (SHOWTIME v2.60 onwards) software development has been in a W95 MSDOS

window on my new Pentium box; the multi-tasking environment of W95 does work,

but some RS232 inputs are clobbered depending on what else the machine is doing.

This has been a blessing in disguise since it allowed me to find a number of

places where the software needed "sanity" checks on data read from the GPS

receiver. Extensive work (culminating in v2.70) has (hopefully) improved the

ruggedness of the code when running on small PCs or in a Windows environment.

-----------------------------------------------------

USING SHOWTIME -- THE MAIN "HELP" SCREEN:

Now let us discuss some of the operating commands in SHOWTIME. When you hit the

SPACE key, you will see the POS window replaced with a "help" screen telling

about the single-keystroke commands. You can use any of these commands from

either the Help screen or while SHOWTIME is operating normally. The escape <esc>

key removes the Help screen and restores the normal display.

[C]OLORS:

You can customize the screen to your preference with the [C]olors menu key.

SHOWTIME assumes only the most minimal IBM-PC video configuration which can

be used on a monochrome or CGA monitor. Hence for the colors, you only have

16 choices. The [C]olor screen lets you assign any of the 16 colors to 10

different screen features. When you [U]pdate the xxx.GPS file in the [F]iles

menu, the latest color choices are saved. I have provided you with default

colors that look nice on my VGA CRT.

[N]OISE (TICKS):

I have often found it useful to have some audible feedback when manually

setting another clock (like the Mark-3 VLBI formatter or my wrist-watch).

SHOWTIME can make "WWV-like" ticks. Hitting the [N]oise key toggles you

through a 3-step sequence: Ticks off / Ticks ON / Ticks ON plus a long tick on

some pre-defined second during the minute. You can select the second for the

long tick in the [G]PS menu. You can also define the pitch of the tick tones

in the *.GPS configuration file. The default tones are 1000 Hz for the seconds

tick and 1200 Hz for the long minute tick.

Note that the time ticks and the screen update are triggered by the 1 PPS

output from the receiver which is sent to SHOWTIME on the RS232 port. If you

set a [T]iming [E]poch offset of 500 msec, the tick souds (and the screen

update) will occur half-way through the second, out of phase with the ticks

you would hear from a radio listening to WWV.

[S, @, D, H, 0-9]: SHOWTIME allows you to set your PC clock to the GPS rec-

eiver either manually or automatically. At any time, hitting [S]et PC Clock

will set the PC's clock with an offset specified in the *.GPS file. Hitting

[@] will enable/disable an automatic PC clock updating at pre-determined

intervals. The keys [1] [2] .... [9] set the automatic update period to be

1.2...9 updates/day. The [D] key sets it one update per day. The [H] key

sets it one update/hour. The [@] key toggles the automatic updates on/off.

The [0] key disables automatic updates (and therefore the [@] key toggle is

disabled). The status of these switches is shown on the [SPACE] help screen.

In case an automatic or manual setting operation wants to set the PC clock

at the top of the minute (between seconds 58 and 02), the setting operation

is deferred for a few seconds since the minute is a busy time for SHOWTIME.

The status of the AutoSet configuration is saved in the xxx.GPS configuration

file in a line that reads "AUTOSET xxx" where xxx is the AutoSet period.

If xxx=0 then the AutoSet operations are disabled until enabled interactively.

All PC clock settings use the TZ: parameters read from the *.GPS file to

determine the appropriate clock offset from UTC.  The TZ: parameters do

nothing else in the operation of SHOWTIME and the actual time on the PC's

internal time clock is not used by SHOWTIME.

AutoSet does nothing to set the PC clock's date, so it is possible to set

The PC's clock and lose/gain one day. Sorry, but the logic to set the PC

date was beyond what I wanted to tackle at this time!

THE SCREEN TOGGLE SWITCHES -- [\], [/], [!], [=], [M], and [L]

In addition, there are some other single-key on/off toggle switches which

can be used interactively to change some screen display parameters.

The [\] key turns the POS screen updating on/off and [/] turns the SAT

screen on/off. These are useful if you want to copy down some numbers from

the screen and freeze the display. Also, if you [R]efresh Screen with SAT

and/or POS turned off, then those portions of the screen are blanked out.

If the [\] and/or [/] switches are turned off (so that the sub-screens are

not displayed), the switch positions are saved in the xxx.GPS file as lines

with the words POSOFF and/or SATOFF so that the program can resume operation

in the previously defined mode. If the lines POSOFF and/or SATOFF do not

appear in the xxx.GPS file, SHOWTIME defaults to having the POS and SAT screens

enabled. On the main help menu, a + or - appears following the [\] and [/]

menu entries to show you if the appropriate screens are enabled.

The [!] key allows you to toggle the main time display between UTC and your

Local time (as defined by the LTZ: parameters in the *.GPS file). The position

of the [!] switch is saved in the *.GPS file in a line reading "TimeSwap UTC"

or "TimeSwap anythingelse". If the TimeSwap line is omitted from the *.GPS

file, the default is "TimeSwap UTC".

The [=] key toggles the display of the current timing offsets (in the top

right-hand corner of the screen) on and off. The main reason for this switch

is to reduce the screen "clutter". The [=] switch makes no changes in the

operation of SHOWTIME and the current timing parameters continue even if they

are not displayed. When you are operating in zero-D timing mode, the [=] key

defaults to ON, and to OFF when operating in 2-D or 3-D positioning mode.

The status of the [=] switch is not saved in the *.GPS configuration file.

The [M]aidenhead Grid Square/G[M]ST key toggles a "ham" feature -- the GMST

field is replaced by the "Maidenhead" grid locator for the current average

Lat/Lon. A typical grid locator might be like FM19me. The first letter is

the longitude modulo 20 degrees measured east from 180 degrees and the second

letter corresponds to the latitude north from -90 degrees modulo 10 degrees.

The next two digits are the Longitude modulo 2 degrees and the Lon/Lat with

quantization of 5/2.5 arc minutes. I have augmented the conventional grid

locator in SHOWTIME with two additional digits for Lon/Lat with 30/15 arcsec

resolution. Thus the "augmented" field is shown with a format like FM19me.75,

corresponding to my home QTH at Long=76d56.098'W and Lat=39d11.292'N. The 7

means that you are 70% of the way in the F1m Longitude band and the 5 means

50% of the M9e Latitude band. The status of the [M]aidenhead/G[M]ST switch is

not saved in the *.GPS configuration file and the program initializes with the

switch in the G[M]ST position. (Just for information, Trimble has included

Maidenhead Grid Square locator readout in their "Scout" handheld GPS receiver

for the amateur's convenience, and I wouldn't allow SHOWTIME to have any

lesser capabilities!)

The [L] key toggles on/off an audible alarm (revised in SHOWTIME V2.30) that

beeps when the GPS receiver has lost lock on all satellites. The unlock

condition is triggered when the receiver has been unlocked for more seconds than

the Lock Hysteresis parameter (initialized by the *.GPS file, and which can be

changed in the [G]PS Mode screen). When this happens, a large flashing "L" char-

acter appears in each of the 6 digits of the time display and beeps once/second.

In addition, the number of seconds during which the receiver has been unlocked

and the "valid since" Date/Time (the last time the receiver achieved full lock)

appear on the [A]veraging parameters screen. When operating in time-keeping 0-D

mode, the "Valid since" date and # Unlocked counter are shown continuously in

the lower left-hand quadrant of the screen. The visual "L" indicator is always

activated and the [L] menu key can be used to toggle the noise on/off.

-----------------------------------------------------

GPS RECEIVER OPERATING MODE CHANGES -- THE [G]PS SCREEN:

Many of the basic PVT-6 operating parameters are set in the [G]PS menu. Single-

key commands optimize the receiver for [p]osition determination or [t]ime-

keeping functions.  [Note that the [p] and [t] commands are case sensitive;

the uppercase [P] and [T] commands take you to the [P]osition and [T]iming

setup screens.]

The PVT-6 has the ability to run in the [A]ll-in-view mode (all channels that

are tracking satellites are used in a least-squares determination of time/

position), or in a mode where it makes use of only the [B]est-of-4 satellites

("Best" determined by the lowest possible xDOP value). Since both position and

timing accuracy are improved by letting the receiver average over the maximum

possible number of satellites, I can think of no logical reason to use the

[B]est-4 mode, but it is included since Motorola allowed for it.

The PVT-6 can make its decision about which satellites to devote its 6 channels

to on the basis of a DOP (dilution of precision) calculation. It can optimize

based on General criteria (GDOP, based on 3-D position and timing accuracy),

3-D position (PDOP), Horizontal position (HDOP), Vertical position (VDOP) or

Timing (TDOP). If you are doing position measurements, I suggest you use PDOP.

You [S]elect the DOP type using the [S] key. Repeated pressing of [S] cycles

thru the various options. If you select the [p]osition or [t]ime options, the

selection is made for you automatically. If you select on the basis of xDOP,

make sure that the [E]levation mask is reasonable since the xDOP criteria want

to include some low-elevation satellites which might not be visible depending

on you local elevation obscurations.

Alternatively, you can also allocate channel resources based on [H]ighest-in-

Sky criteria with the 6 channels allocated to the 6 GPS satellites with the

highest elevation -- particularly suitable for accurate [t]imekeeping. If you

select [H]ighest, SHOWTIME selects a 1� [E]levation mask automatically, but

this can be changed with [E]levation if you want to set a different limit.

The [%]APtype switch is included for experimentation. Motorola describes it

in their documentation. I have found it of little use in a non-dynamic

environment and suggest you use the default Aptype 4=Fixed position. Beginning

with SHOWTIME V2.20, [%]APtype is a "rotary switch" -- press the [%] key

repeatedly until the desired value appears.

In SHOWTIME 2.50 a new switch, [G]PS Ignore/Use, has been added to allow you

to selectively ignore any of the PRN01 thru PRN31 satellites. After hitting

[G] you will be asked for a GPS PRN #, and that satellite will be toggled

between on/off. Responding with a "0", or hitting <cr> will unconditionally

enable all satellites. The screen shows PRN31 thru PRN01 (in clusters of

eight with PRN31 on the left-hand end) with either a "+" or "�" to show

on vs. off. The "�" symbol for disabled satellites also appears next to the

PRN in the satellite status display in the lower right-hand quadrant of the

screen. The Ignore feature only works for PRN's 01 thru 31 (i.e. NOT PRN32)

because I was too lazy to solve a problem withe Microsoft's long integers.

-----------------------------------------------------

POSITIONS and AVERAGING -- THE [P] and [A] SCREENS:

The POS window displays the fixed "REFerence" position, an average position and

the most recent instantaneous position, all defined in the right-handed NEU

(North/East/Up positive) WGS84 coordinate system. The differences (REF-AVG),

(AVG-LAST) and (REF-LAST) and their RMS values are given in meters (where 1� of

Lon=111.111km*cos(Lat) and 1�Lat=111.111km). All SHOWTIME heights are referenced

to the WGS-84 ellipsoid, and NOT to local mean sea-level (MSL). The WGS84 vs.

MSL difference can be as much as about 50 meters. When running in the Zero-D

(timing) mode, only the fixed reference position is displayed on the screen

since the receiver is not determining any positions.

The avgerage positions computed in SHOWTIME are NOT weighted with the GPS xDOP

(dilution of precision) for simplicity. Both the average positions and their

RMS deviations are smoothed by a filter that works like like:

AVERAGE = [(Filter Length - 1)*oldvalue + newvalue] / (Filter Length)

Initially, the filter is set = 1, and then it is incremented by 1 per second. Simple

algebra shows that the average thus computed is the running average. When WEIGHT

hits the defined filter length "clamp" value, the incrementing is stopped. When

the filter length reaches the "clamp" value, the Averages are then exponentially

weighted with a time constant related to the "clamp" value. If you are running

with PCRATE > 1, then the receiver sends position data at a lower rate, so the

running filter length is incremented by 2 or 3. See FAQ #23 for some more

details on this new capability.

With Version 2.85, a new capability to weight the average position/rms with

the HDOP/VDOP (Horizontal/Vertical Dilution Of Precision) for the actual

satellite geometry. The [A]veraging [D]OP Weighting switch toggles the DOP-

weighted averaging on/off. The [D]OP Weighting switch can be set from and

saved to the *.GPS configuration control file.

When SHOWTIME is running, hitting the [A]verage key will display a small menu

that allows you to manipulate the averaging parameters. You can reset the

[r]ms or [p]osition or [t]ime counters to start a "fresh" average and you

can zero the [U]nlock counte, or you can zero all four with the [z] key.

Note that the [p], [r] and [t] "zeroizer" keys are all lower case.

You can change the "clamp" time constant for the [P]osition, [R]MS or [T]ime

averages. When you enter a new "clamp" time constant, you can simplify the

entry by using the "hint" units shown in the menu -- for example, 2H sets the

averaging period to 2 hours = 7200 seconds. Note that the [P], [R] and [T]

commands must be entered in UPPER CASE. When you [F]ile [U]pdate, the *.GPS

configuration file in the [F]iles menu, all the current "clamp" filter time

constants are saved and can be restored the next time you run SHOWTIME.

For really accurate time-keeping, the GPS receiver needs to be constrained to a

fixed position, which should be accurate at the 10-15 meter level. The position

that is used for this is called the REFerence position. Also, if you chose to

operate in the 2-D (height fixed) mode, the receiver fixes the height to the

REFerence height.

When you begin operation at a new location, you will probably not know your

position accurately. The raw positions reported by the GPS receiver will wander

by up to �100 meters horizontally and �250 meters vertically because of weak

geometry of the satellites, errors in the broadcast ephemerides, atmospheric

propagation errors and especially because the US Military degrades GPS by

dithering the spacecraft clocks (known as SA = Selective Availability).

To overcome these errors, it is desirable to average position measurements for

several hours to get an accurate position. When the receiver has been running

in position mode for a while, you can use the [P]osition menu to transfer the

average position into the REFerence position. The [P]osition menu also allows

you to manually enter coordinates. You should also use the [R]eference ID

command to enter a brief (36 character max) text description describing the

setup conditions. The Reference ID appears on the bottom of the POS screen

(unless an RTCM SC-104 DGPS beacon is being received, in which case the DGPS

information is shown on the bottom line). If you make any Position changes, the

Averaging filters are zeroed so that the position averages will be "fresh".

SHOWTIME assumes all position coordinates are in the GPS WGS84 reference frame

(which is insignificantly different from the ITRF reference frame used by the

geodetic science community). The main confusion about WGS84 comes in the defin-

ition of heights. Conventional (old) topographic maps and civilian applications

have historically referred heights to Mean Sea Level (MSL) which differ from the

WGS84 global reference geoid by up to about 100 meters (in the Washington DC

area, MSL heights are about 34 meters greater than WGS84 heights -- i.e. local

MSL corresponds to a WGS84 altitude of about -34 meters on the USA east coast).

When running in 3D position determining mode, SHOWTIME gets its height inform-

ation from the $GPGGA NMEA message which reports both the height and the Geoid

reference offset. The NMEA messages are intended for maritime navigators and the

$GPGGA height is (supposed to be) referenced to MSL. In May, 1995 I stumbled on

the rude discovery that Motorola's ONCORE firmware releases 5.x/6.x (pre-1995

firmware) incorrectly reported the $GPGGA height in WGS84 coordinates; since the

intital Totally Accurate Clock and SHOWTIME development was done with 5.x/6.x

receivers, early versions of SHOWTIME simply ignored the geoid correction.

SHOWTIME was always intended to always run in WGS84 coordinates for time-

keeping purposes. The position you enter as the REFerence position is loaded

into the receiver in WGS84 Geoid units. The positions displayed were intended

to be WGS84 positions.

[Note: The AVGPOS.BAT utility I have provided uses Motorola's GPS53

controller software, not SHOWTIME and produces WGS84 coordinates.]

To my surprise, in mid-May '95 I received the first ONCORE receivers with their

firmware release 8.0 (which shows a release date Jan.11, 1995 -- use the

VERSION.BAT utility to verify what version you have). In version 8.0, Motorola

has corrected the earlier NMEA GEOID error. This caused me to revise SHOWTIME

to accommodate the old vs. new firmware differences.

I have added a new switch called "GEOID" in the SHOWTIME V2.20 code. If the

switch GEOID=0, then no geoid correction is applied, as appropriate for old

Motorola 5.x and 6.x firmware releases. If the switch GEOID=1, then the geoid

correction is applied to reported heights as required with new Motorola 8.x

firmware. The switch appears as GEOID 0 or GEOID 1 (0=off, 1=on) in the *.GPS

configuration file and it can be toggled on/off with the [W]GS84 GEOID key in

either the [P]osition or [G]PS Receiver screens (which also show the words "old"

and "new"). The current GEOID switch value is saved when you update the *.GPS

control file on the [F]iles screen. If GEOID is not specified in the *.GPS file,

the default is  GEOID 0  for compatibility with earlier SHOWTIME versions. The

position of the Geoid switch is also displayed as "Geoid:N" or "Geoid:Y" in

the "Rcvr:" status line in the lower-left hand quadrant of the screen. Both the

WGS84 and MSL heights (which differ by the value of the Geoid offset) are shown

in the [P]osition screen, with the sign of the correction depending on the

current position of the GEOID on/off switch. [See also FAQ #7 for more detail]

-----------------------------------------------------

TIMING FUNCTIONS -- THE [T] SCREEN:

Finally, we come to the real reason for doing the "Totally Accurate Clock"

project -- producing high accuracy timing data. Repeated tests of the "TAC"

with respect to a Hydrogen Maser show that the 1PPS tick from the PVT-6 has

a precision of 45 nsec or better when the receiver is running in a mode

"[t]iming -- best parameters" command (set in the [G]PS Mode menu) and with an

accurate reference position).

Two early independent tests (one by MOTOROLA in cooperation with the US Naval

Observatory, and one by me comparing the prototype "TAC" against a USNO

traveling clock) showed that the PVT-6 has an intrinsic accuracy of �20 nsec

with respect to the USNO master clocks -- providing that you account for all

the instrumental effects! Subsequent tests I have run confirm the earlier

traceability to the USNO master clocks with several "production" "Totally

Accurate Clocks". [See FAQ#14 at the end of this document for details]

One of the instrumental biases that must be included is the propagation delay

in the coax cables connecting the GPS antenna to the receiver and the cable

connecting the receiver's 1PPS output to whatever is the "time sink". Both

cables make the delivered pulse be LATE and so the receiver must be advanced

by a corresponding amount.

Another instrumental bias that needs to be removed is any delay added in the

receiver system. In the "TAC", we buffer the 1 PPS output through a pair of

high-speed 74AC04 logic gates (my prototype used 74HC14's with a bit more

delay) which further delay the pulse. For the 74AC04 gates used in the TAC

package, the appropriate instrumental delay is �8 nsec (SHOWTIME default).

In Motorola's GPS53 OEM controller software, these delays are compensated with

the PPSDELAY command. In SHOWTIME there are [T]iming commands to account for

[C]able and instrumental [D]delays separately. For both these SHOWTIME commands,

you can enter the number followed by suitable units character(s).

For example, you can enter the [C]able delay as either 50 FT or 25 METERS and

SHOWTIME will compute the appropriate number of nsec for a 50 foot or 25 meter

cable length (assuming a velocity factor of 0.66 for typical coaxial cables).

On any of the [T]iming commands, you are told what are the default units, and

you are given a "hint" of the alternative units you can use.

In many applications, you will want to offset the tick epoch from the UTC

second. The ONCORE PVT-6 (with Option A) allows 3 ways to do this offset:

- The basic measurement epoch of the receiver can be stepped in 1 msec

steps. (In the MOTOROLA GPS53 controller software, this is done with

the EPOCH command.)

- The pulse can be moved LATE with respect to the receiver epoch in 1

nsec steps from 0 to 999999999 nsec. (MOTOROLA GPS53 command PPSOFF.)

- The pulse can be moved up to 1 msec EARLY with respect to the epoch in

1 nsec steps (MOTOROLA GPS53 command PPSDELAY).

The bookkeeping on these 3 numbers can lead to much confusion, which I have

attempted to remedy in SHOWTIME. Parameters for setting the time epoch are

all handled in the [T]iming menu.

- The [C]ABLE delay and INSTrumental [D]elay offsets can be entered

separately. (These are used like MOTOROLA's PPSDELAY inputs.)

- You enter separately a nominal msec coarse [E]poch offset and a de-

sired fine [u]sec offset (the letter [u] is used since the Greek � is

not easy to type on a PC). The fine �sec offset can be either earlier

or later than the msec [E]poch and the software will take care of the

arithmetic signs for you (The computed values are used like MOTOROLA's

PPSOFF and EPOCH parameters). You can also [S]lew the current values

either earlier or later or [Z]ero both the [E] and [U] offsets.

- You can offset the tick early by a small [I]ntentional offset which is

not cleared by the [Z]ero command (This is applied like the [C]able

and Instrumental [D]elay parameters equivalent to Motorola's parameter

PPSDELAY). The purpose of this extra offset is to allow you to have

the GPS tick occur earlier than the "house" time standard to facilitate

unbiased time-interval measurements. While you are in the [T]ime setup

screen, the display shows the total [E]+[U]+[I] offset from UTC.

- Back on the main CLOCK screen, the 3-digit msec field (shown in large

digits at the end of the time field) is the nearest msec epoch to the

tick time. The "1 PPS offset" field shows the TOTAL offset (+late, - early)

of the tick from the UTC second.

In SHOWTIME I separated these various contributions and do all the arithmetic

for you because in the VLBI world I have often seen time synchronization

problems because of sign confusion when several contributions are combined.

Another parameter that is displayed on the main screen is my estimate of

the current timing accuracy. The basic algorithm for estimate is

Timing sigma = �[��+(��/N)]

where N is the number of "extra" satellites contributing to the timing:

N = (# Satellites being tracked)        for the zero-D timing mode,

or  N = (# Satellites being tracked - 2)    for a 2-D position fix,

or  N = (# Satellites being tracked - 3)    for a 3-D position fix,

and where my experience shows � � 10 nsec  and  � � 100 nsec seem appropriate.

In addition to the current Timing sigma in the upper-right hand part of the

UTC time field, the maximum, minimum and average values of Timing sigma are

shown at the bottom of the POS screen. The averaging parameters are set on

the [A]verage screen.

-----------------------------------------------------

FILE UPDATES -- the [F] COMMAND:

At any time you can hit the [F]ILE command. In the [F]ILES screen, you are told

if there have been any major changes to the screen colors, receiver operating

parameters or epoch timing parameters. You can [U]PDATE the current receiver

configuration data file so that you can resume operation later with the same

setup. If you change the [F]ile name, then you Update to a different file. If

you select a [F]ile name that already exists, you are given the option of over-

writing the existing file. The old file name is changed from *.GPS to *.GP$ as

a backup. The file you write with the [U]pdate or [F]ile command is a complete

snapshot of the important receiver and software parameters.

You can [L]oad a different configuration file. All parameters that are defined

in the file you [L]oad will override the existing values and the file name for

the next [U]pdate is changed to the file you just [L]oaded.

In the file screen you can do a few DOS operations like  [D]IR *.GP?  to get a

listing of the *.GPS and *.GP$ files in the active directory and [C]hange to a

new directory. You can also  [S]hell out of SHOWTIME to execute DOS commands.

When you [S]hell to DOS, you lose about 140 kbytes of memory since the SHOWTIME

code is still resident. If you change directories while in the DOS [S]hell,

SHOWTIME will use the new directory for any subsequent file read or write

operations. To return to SHOWTIME from the DOS [S]hell, type EXIT.

-----------------------------------------------------

EXITING SHOWTIME -- THE [X] COMMAND:

From the main menu (or from the [P]osition, [T]iming [F]ile and [G]PS screens)

you can type [X] to exit SHOWTIME. You are told if there have been any parameter

changes since the last update and you are asked TWICE to confirm that you want

to exit. Responding with two "Y" answers will terminate the program. Responding

with an "F" will take you to the [F]ile update screen. <esc> will abort the exit.

If you abort the e[X]it with <esc>, the receiver will re-synchronize the RS-232

communications and resume normal operations.

If you have a glitch in the the receiver's power or the RS-232 communications,

SHOWTIME may abort with an unrecoverable I/O error message and return to MSDOS.

This is a Microsoft QB45 error message. I have tried to trap this error without

having SHOWTIME abort, but I have been unsuccessful. Sorry, but this quirk has

to be tolerated until SHOWTIME is ported to a "real" computer language.

--------------------------------------------------------------------

FAQ's -- IS THE GPS RECEIVER LOCKED UP?  IS IT WORKING?  HOW ACCURATE IS IT?

I have received number of questions (especially for the NASA/GSFC versions of

the "Totally Accurate Clock") on receiver performance, so let me answer some

FAQ (Frequently Asked Questions) here,  en masse:

----------------------------------

(1) The SHOWTIME program seemed to start OK on my TAC. The initial signon

messages appeared, and then the initial "AM I CONNECTED? test" sounded beeps

and showed a pattern like .........#..#..............#..#.......  -- but then

nothing happened. What's wrong?

In this case, the user had an old RS232 port on his computer, and it was

equipped with a 25-pin RS232 connector. The SHOWTIME initialization routine

ONLY looks for the presence of a 1PPS signal on the DCD wire, but does not

check for other RS232 data signals. Their 9/25 pin adaptor widget had pins

2 and 3 swapped. The correct wiring for the cable is:

TAC RS232                  PC RS232   -or-    PC RS232

(9-pin)                    (9 pin)           (25 pin)

DATA gnd      5-------------------------5-----------------7

TXD to GPS    3-------------------------3-----------------2

RXD from GPS  2-------------------------2-----------------3

DCD input     1-------------------------1-----------------8

Note that only the 4 wires shown in this sketch are used in the production

TAC's and all other wires are ignored and un-needed.

----------------------------------

(2) Occasionally I see excursions of several microseconds between the TAC

output and a good frequency standard. Why?

In ALL my testing I have never seen any spurious timing pulses generated

except when the receiver becomes disconnected from the antenna or when RFI

is present and no GPS satellites are being tracked! Therefore I suspect your

antenna connection was faulty in some way. When you have no GPS signals being

received, the 1PPS from the ONCORE is generated by crystal oscillator running

at the rate last steered by GPS. Its drift rate depends on your operating

conditions, but seems to be a few hundred nsec/minute typically.

In version 2.10 (logic revised in version 2.30) of SHOWTIME I have helped

this problem -- I added a "loss of signal" unlock indicator that warns you

when the receiver has lost lock. When this happens, each digit of the UTC

time displayed on the PC is over-written with a large flashing "L" (in the

color defined by the warning parameter ColorRED).  The total number of

seconds that the fault has occurred appear on the screen when you are in

[G]PS [t]iming mode and also in the [A]veraging parameters menu screen.

The counter of the total seconds of data loss is reset by blipping the

[A]veraging [t]ime or [A]veraging [z]ero all 4 controls. This obnoxious

alarm can be toggled ON/OFF with the [L] key but it defaults to being turned

ON whenever SHOWTIME is started. The flashing "L" on the screen and the

audible alarm are triggered whenever the receiver has been unlocked for

a time longer (in seconds) than defined by the [L]ock Hysteresis parameter

which is initialized in the *.GPS file and by the [G]PS Mode [L]ock Hyst

parameter. Note that LockHyst is set negative to silence the audible alarm.

The default is (positive=alarm noise on) but the audible alarm can be

defeated by the main help menu [L] key.

If the fault is persistent, then I'd suggest you first check the TAC's

antenna connector and verify that it has the +5VDC bias voltage present.

If the cable developed a short, then it is likely that the fuse inside

the TAC has blown and nothing will work including the front-panel LED.

Also, you should look for possible RFI (the old Rogue GPS receivers are

notorious RFI generators, and the newer TurboRogue receivers can also cause

problems). Try relocating the antenna and/or receiver to clear the problem.

----------------------------------

(3) The TAC displays a bogus date and/or time when I turn it on. After �10

minutes it finally starts working OK. Why?

It is likely that the small 9 Volt battery inside the TAC is discharged.

This could be because you have not used the clock for several months. The

battery is a NiCd unit and its main purpose is to keep a low-accuracy

wrist-watch crystal controlled clock on the Motorola ONCORE receiver

running.

In our TAC implementation, the battery is charged at a very low (�1 mA)

rate thru a diode and a 3kohm resistor. The clock requires a few tens of

uA and will discharge the battery after a few weeks of non-use. Applying

power to the TAC will start the battery charging, but it will take several

days to become fully charged. If this slow charge rate causes you problems,

you can increase the charging rate to �10 ma by putting a 470 ohm resistor

across the 3kohm resistor supplied with the TAC.

If the GPS receiver wakes up with a "dead clock", it will go into a

"sky search" mode and start looking blindly for GPS satellites. As soon

as it acquires the first satellite (which may take 5-15 minutes), it will

get an approximate time synch that is good enough to continue operation.

The ONCORE receiver stores GPS "almanac" data (low-accuracy Keplerian

elements) in non-volatile RAM, so as soon as it knows the current time

and an approximate position, it can begin searching for satellites that

it knows should be visible. Even if the receiver has lost this data, the

complete set of almanac data for ALL the GPS satellites is broadcast by

each GPS satellite every 12.5 minutes. Hence a completely "cold" ONCORE

receiver should take no more than about 15 minutes to begin working.

Beginning with SHOWTIME V2.20, I have added a "Coldstart The Clock" option

in case you have trouble with a receiver with a dead battery. Go to the

[P]OSITION screen where you will find an entry [D]ate/Time Coldstart.

Simply enter the current UTC date and Time. The time should be accurate

to about 10-15 minutes. If you miss the date by a day or two, don't worry.

When the receiver locks up on the first satellite, it will correct the UTC

time and date in a few seconds. If you use [D]ate/Time and the receiver

already has a valid date/time, the receiver simply ignores the request.

----------------------------------

(4) Occasionally the time display misses a second and then jumps ahead by 2

seconds. The WWV-like [N]oise ticks also skip a beat.  It is most "ragged"

when the satellite status display shows 9 or more satellites are visible

above the elevation limit. Why?

It takes 600-800 msec for the ONCORE receiver to send all its 4800 baud

NMEA messages. The length of time depends on how many satellites are in

view because each $GPGSV message reports on only 4 satellites -- hence

with 9 satellites, GSV messages are required in addition to the other

3 messages that the receiver sends ($GPZDA, $GPGSA, $GPGGA). The SHOWTIME

software has an internal timer (called XTIME) that paces when it is

listening for messages. The software then does some display updates and

computes the next second's display, and then waits for the 1PPS tick to

arrive on the DCD RS232 wire.

You can change the XTIME slice from the default value in the [G]PS screen

with the [^] key. If this doesn't fix the problem, then slow down the NMEA

message rate with the PCRATE parameter on the [G]PS screen. I find that

an XTIME of 0.85 (850 msec) works fine for me on my personal 486/66.

----------------------------------

(5) The bar-graph display shows the Signal-to-Noise Ratio (SNR). What are the

units for the SNRs?  What are "good" values?  What range should I see?

Motorola returns the SNR in the $GPGSV messages in units that are nominally

equal to dB. The maximum value the ONCORE reports is 36dB and the SHOWTIME

bar-graph display can accommodate SNRs up to 38 dB.

If you are using an antenna with a really good noise figure which is in the

open (no trees to attenuate the signal), you will probably see satellites

at high elevations showing 33-36 dB SNR's. The small Trimble and Motorola

patch antennas will probably show about 30 dB maximum SNR since they are

more lossy and have poorer system temperatures.

Since the bar-graph "S-Meter" is sorted by satellite elevation, you can get

a good feeling for the gain pattern of your antenna. The amplitudes should

not bounce around more than � 1 dB from second to second unless you have

high multipath or spotty obscuration by trees. Since the "S-Meter" display is

easy to read and has a fast 1 second response, I have used SHOWTIME as a "GPS

OHMMETER" to make certain that another GPS antenna/receiver is working OK!

Motorola provided little information about units it uses for SNRs, so let me

give my speculations. The GPS signal structure uses a 1.023 MB/s pseudo-random

CDMA (code-division multiple-access) spreading code. Each operational GPS

satellite has a unique 1023 bit PRN (1 msec period) called the CA (Coarse

Acquisition) code -- in essence the satellite serial number. These CA codes

("Gold Codes") were selected to have maximum mutually orthogonality. The CA

codes are made by convolving two different 10-bit shift-register sequence

generators. Two different taps on one shift-register (with 10 choices for

the position of the first tap and 9 choices for the 2nd, resulting in 45

unique codes) and the output of the second 10-stage generator are multiplied

in a 3-way XOR gate to generate the 45 possible codes. GPS reserves 32 PRNs,

several more are reserved for pseudolites, and the rest of the 45 are unused.

[Note: If anybody wants to study these codes in more detail, I have included

a small QUICKBASIC program called GPS_CODE.BAS in MISC.ZIP for your use.]

The GPS digital data has a bit rate of 50 bits/second, i.e. it takes 20 CA

code repetitions to convey one bit and the matched-filter data bandwidth is

hence 50 Hz. Therefore the signal "spreading gain" = 10*log(1.023Mb/50b)

or 43 dB for each satellite. The downlink signal from the strongest GPS

satellite is about half the noise in the 2 MHz (approx) passband of a simple

CA code GPS receiver. Therefore the best SNR=S/(S+N) we can expect is about

38-39 dB. If there are several satellites in view, each receiver channel

sees all other satellites as uncorrelated noise, so the 36 dB SNR "clamp"

that Motorola set is a reasonable, practical upper limit to the SNR.

Since the 1023 bit CA codes are of finite length, we expect some random

correlation with a one-sigma dynamic range of about 10*log(1023) � 30 dB.

The correlators in typical GPS receivers are one-bit devices (like those

used in VLBI) so they suffer about 2 dB (the normal �/2 Van Vleck factor)

in SNR loss, with the lost power appearing as code-to-code cross-modulation,

further decreasing the dynamic range. The practical maximum (i.e. several

sigma) dynamic range for GPS is in the 23-25 dB range. If the strongest GPS

satellite has SNR = 33 dB, then the weakest detectable satellite will have

an SNR of about 10 dB (assuming Motorola's numbers are really dB).

----------------------------------

(6) The time displayed in SHOWTIME "sticks" at a strange value, and this

seems to repeat. The only way I can get it to work is to restart SHOWTIME.

"The best laid plans of mice and men aft gang agaly"  --  this problem arose

because of a bizarre "feature" Motorola decided to implement which I had

ignored. When running in zero-D "timekeeping" mode, the time field in the

$GPGGA message does not get updated. The only valid time-of-day is in the

$GPZDA message. Versions of SHOWTIME prior to 2.20 took the time from

whichever GGA or ZDA arrived first, providing it was larger than the

previous report. Hence the clock display could "stick" at the invalid

time reported in the GGA message in zero-D mode if the GGA messages

happened to arrive each second before the ZDA message. The bug is now

fixed in 2.20 (I hope -- if you see it happen, let me know!).

----------------------------------

(7) A new switch GEOID is in the V2.20 code. What is it? Why have my reported

heights changed?  My newest (1995 production) ONCORE receiver seems to report

different heights than my earlier receivers. Why?

All these seemingly unrelated questions have to do with the WGS84 reference

system for positions and a change Motorola seems to have made in their ONCORE

firmware in early 1995 (Version 8.x). See the new discussion in the POSITIONS

AND AVERAGING section earlier in this document. The bottom line is:

- If you have an older ONCORE receiver (Version 5.x or 6.x code dated prior

to Jan.11, 1995), then set the SHOWTIME GEOID switch off (set GEOID 0 in

the *.GPS file or use the [W]GS84 switch on the [G]PS/[P]osition screens).

- If you have a more recent ONCORE receiver (Version 8.x firmware

dated Jan.11, 1995 or later), then set the GEOID switch on (set

GEOID 1 in SHOWTIME.GPS or use the [W]GS84 switch).

- If you want to verify your software release version/date, fire up

the Motorola controller software (GPS53), hit [F8] to run a self-

test, then type "id" to have the receiver dump its configuration

information. When you finish, type "mode fix". Or you can run the

utility I have provided called "VERSION.BAT" (which uses EXC.EXE

to run the Motorola controller software and issue the necessary

commands to determine the version with minimal intervention).

All this confusion applies ONLY to the units for the height when you are

running the receiver in the positioning averaging mode in SHOWTIME. The

only effect on the receiver's operations occurs if you let SHOWTIME average

the positions and the GEOID switch is wrong, you will get a bad Reference

height, which will introduce a timing bias error when you run in zero-D

timing mode with the bogus reference height -- see FAQ #14 below for a

discussion of the magnitude of the error. The error (Geoid separation) is

�34 meters on the east-coast of the USA, corresponding to a timing bias

of about (34 meters)/(c = 0.3 meters/nsec) * 63% � 70 nsec.

[Note: If you generate the REFerence coordinates using the AVGPOS.BAT

utility I have provided, the positions in the POSITION.GPS file are

are guaranteed to be in the correct WGS84 reference frame regardless of

the setting of the GEOID switch.]

----------------------------------

(8) I have an older PVT-6 equipped with "Option A", but SHOWTIME doesn't

work with it. Why?

Motorola made some significant code changes beginning with their 5.0

release. Their 4.x receivers (and presumably earlier ones also) did not

implement the $GPZDA message (that reports the date & time in all modes)

and $GPGSV (reporting on the visible satellites and their SNR). Also,

their code versions prior to 5.0 had a �560 nsec time bias error, so they

are not recommended for precise timing use. Contact Motorola to arrange

for a firmware update! Versions 5.0 and later all seem to work similarly.

----------------------------------

(9) Will SHOWTIME run with other GPS receivers (i.e. not ONCORE with Option A)?

Probably not. Many of the receivers do not generate all four of the NMEA

messages that SHOWTIME uses. SHOWTIME requires that a �200 msec 1 PPS pulse

is sent by the GPS receiver to the computer on an RS232 status line. Someday

I may write a similar, but more general GPS time display program.

----------------------------------

(10) The UNLOCK counter on the [A]veraging screen shows a few counts each

day even though the "Totally Accurate Clock" seems to be working OK. Why?

The receiver can unlock for 1-2 seconds when it begins tracking a new GPS

satellite after an old one sets. It can "glitch" briefly if there is local

RFI. (See FAQ#2 above) Don't worry if you see a few counts.

If you start seeing 100 counts or more, something is definitely wrong! You

should look for possible problems in the coax cable feeding the GPS antenna

(connectors are always a likely culprit). Perhaps your power supply is noisy

or the DC power connector is loose. Wiggle everything to find the problem.

----------------------------------

(11) Regarding the satellites reported as "Locked" and "Used (*)" -- sometimes

a satellite is Locked but not Used? Why?

When a satellite is added "fresh" to the receiver (it just came above the

elevation limit, or now deserves to be in the "HIGHEST IN SKY" list, or it

now needed to make the optimum xDOP), the satellite must be tracked long

enough to decode a complete set of ephemeris data. A GPS satellite sends it

own precise ephemeris every 30 seconds, and you have to wait for a complete

block to be decoded. If a satellite is still weak, the carrier phase and CA

code tracking loops may be locked, but some bit errors occur during the

decoding of the ephemeris, so the receiver must wait for another 30 seconds

to try again to decode a complete 1500 bit message.

Or, you may have the receiver in the "Best-4" mode in which case only 4

satellites are used (not recommended but permitted!).

Or, if you happen to be running with a DGPS beacon correcting the receiver

and the beacon is not reporting corrections on a satellite your receiver

is tracking, then it will not be used.

----------------------------------

(12) I am running in the receiver in the "Highest-in-Sky" mode and a satellite

that should be in the "tracking" list is not there. Why?

This could be for several reasons. First, you could have an obstruction in

the part of the sky where the satellite is located, so it can't be detected.

Or, the satellite in question may currently be flagged as "unhealthy" --

the DoD people who control the satellites can instruct a satellite to report

itself as unreliable. The DoD may do this if they are uploading new commands,

or if they are maneuvering the satellite, or if there is a problem, or

perhaps just because the feel grumpy.      C'est la vie!

Or, perhaps you have added the sattelite to the "ignore" list ([G]PS [Y]es

[G]PS ignore), in which case the satellite's PRN will be followed with a

"�" character on the satellite status screen and will appear in the *.GPS

file in a line reading like "IGNORE  +++++++++++++++++++�+++++++++++|"

----------------------------------

(13) When SHOWTIME starts up, the little up- and down-arrows that show when

the satellite is rising or setting don't appear for a while. Why?

The up/down arrows are generated in SHOWTIME by looking at the elevations

reported by the receiver (quantized in 1 degree steps). When the elevation

report changes, then SHOWTIME can make the rising/setting determination.

SHOWTIME has to wait a few minutes to see the elevation change.

----------------------------------

(14) What is the accuracy and precision of the "TAC" 1 PPS output signal?

First, about the precision: The 1 PPS output from the ONCORE receiver is

derived from a �9.5 MHz crystal. There is no phase-resolver on it, and the

individual tick pulses are derived from this oscillator. You can expect any

single tick to be anywhere (uniformly distributed) in a 104 (i.e. � 52) nsec

window centered on the correct epoch. The pulses will wander in a "saw-tooth"

pattern thru the � 52 nsec window at an indeterminate rate. Experience has

shown that averaging the measurement over a 5-10 second period is needed to

remove noise from this sawtooth. Since the peak-to-peak error is � 52 nsec,

the per-pulse RMS error is about 30-35 nsec (� one-third of the p-p value).

Averaging for 10 seconds then reduces this effect as �N to � 10 nsec RMS.

The DoD "dithers" each GPS satellites Cesium clock in a process called Sel-

ective Availability (SA). The SA modulation spectrum has an RMS amplitude of

�100 nsec RMS (�300 nsec peak-to-peak) with modulation time scales from 1 to

2000 seconds. The SA modulation is not coherent between the different GPS

satellites, so that when you are running in zero-D "All-in-view" time-keeping

mode tracking N=5 or 6 satellites, the SA effects are reduced as �N. Thus the

�100 nsec RMS errors are reduced to the 40-50 nsec RMS level (assuming 5-10

second averaging is used to remove the pulse-to-pulse "sawtooth" jitter). The

RMS precision goes down to �30-35 nsec with 30-100 second averaging times, and

the Allan Variance �T/T scales as � 50 nsec/T, where �T is the uncertainty in

measuring over an elapsed time interval T.

Accuracy is affected by several factors, some of which (a, b, c and d in the

following list) are beyond our control and must be tolerated, and some of

which (e, f and g) we can some control:

(a) The timing effects of SA, plus the (in)accuracy of the satellite ephem-

erides, satellite clock models, ionosphere corrections, etc. that the

DoD chooses to provide us in each satellites's real-time data stream.

(b) The observing geometry -- where are the GPS satellites in the sky?

(c) The "correctness" with which Motorola has implemented their internal

receiver firmware.

(d) Propagation delays of the signals in the earth's atmosphere.

(e) The accuracy of the REFerence station positions that are used.

(f) The accuracy with which we account for all the cable and instrumental

delays in our measurement setup.

(g) Errors caused by multipath at the GPS antenna.

To test the a+b+c+d+e+f+g "bottom line", we have done a number of tests at

the Goddard Geophysical & Astronomical Observatory (GGAO) and AlliedSignal

Technical Services Corp. (ATSC) which have yielded the following results:

- We routinely show a measurement repeatability of 40-45 nsec comparing

a "TAC" against a Hydrogen Maser when we use 5-10 second time-interval

averages, after a simple linear drift is fit to account for Maser

frequency offsets.

- When these results are smoothed, systematic biases at the 10-20 nsec

level are seen over times of hours to days. Assuming that the Hydrogen

Maser is not at fault, these are likely due to (a) GPS ephemeris and

clock model errors, although the (d) propagation delay errors are

undoubtedly also present. (Robin Giffard at HP (Palo Alto) reports

seeing similar long-term "wandering".)

- We compare a "TAC" against the USNO master clocks using a "TV line 10"

time synchronization scheme (calibrated by traveling clocks which go

between GGAO, ATSC and USNO monthly) and by direct comparisons when

the traveling clocks visit us. The USNO-to-TAC offsets are rarely

bigger than 20 nsec. (Mihran Miranian at USNO and I plan future tests

using a TAC at the USNO master clock whenver we get around to it.)

- The most critical REFerence position parameter is the station height. To

test the (e) dependence, I have intentionally corrupted the station

height by �30 meters (equivalent to �100 nsec). With the height error of

+30 meters (HIGH), the TAC pulse was observed to be LATE by 61�3 nsec

(the results being obtained with 1000 second averaging times on an HP5328A

counter with 10 nsec one-shot time-interval capability). When the height

error was set -30 meters (LOW), the pulse was seen to be 64�2 nsec EARLY.

From the average of these tests, we get the "63% rule":

(Timing Error) � +63% * (Height Error)

where a positive time error means LATE, and where the height error is

expressed in nsec. The coefficient is not 100% because of geometric

effects; each satellite contributes to the error as  sin(Elevation) and

the error is averaged over 5-6 satellites and the mean satellite elevation

(at mid-latitudes) is typically �40� and sin(40�) = 0.64.

- As a partial test of the (c) Motorola contribution and on the repeat-

ibility of our "TAC" hardware fabrication, I carefully compared three

different TACs with three different Motorola firmware releases (versions

5.1, 6.0 and 8.0) running from the same GPS antenna thru an 8-port GPS

power splitter and with identical length cables. Again I used 1000-sec

averaging and the three TAC packages showed ZERO differences �2 nsec.

As a test of other possible Motorola receiver "quirks", I have run tests

where the "TAC" is powered down or the antenna is disconnected for

several minutes. In all cases, the receiver recovers in a minute (after

it shows that it has re-acquired the GPS satellites) with no loss in

accuracy/precision and with no discontinuities at the few nsec level.

- Regarding (d) atmospheric biases, the troposphere contributes a zenith

path delay of 1-3 meters (3-10 nsec) which maps thru the receiver as

the equivalent of an error in the station height. Using the "63% rule"

described above, this can induce systematic errors in the 2-6 nsec

range. Motorola does not state if they make any attempt to correct for

tropospheric delay in their documentation so I must presume that they

do not. The ionosphere contributes up to 10 meters of delay at L-band.

See the following FAQ (#15) for comments on [I]onospheric corrections.

- Multipath induced errors (g) are tricky and beyond the scope of this

discussion! The main thing you can do is to make sure your antenna has

a clear view of the sky and that there are few reflecting surfaces

nearby. You can get a clue when you may be having multipath problems

if you see the SNR "bar-graph" bouncing around by more than than 1 dB

over time scales of a few seconds, especially at low elevation angles.

- A number of tests similar to those I have described here were done

by Mike King and David Busch (Motorola) and Mihran Miranian (USNO) in

1993 and are described in the Institute of Navigation (ION) Jan.1994

Conference Proceedings. I have enclosed a reprint of this paper (with

some additional notes on my measurements appended) with all the TACs we

at GSFC are distributing. Their results are quite comparable to mine.

Assuming that you have determined your REFerence station positions to the

5-10 meter level by suitable averaging and that you account for all the

cables in your setup, and that you average over the � 52 nsec "sawtooth",

the "bottom line" is that the ONCORE/TAC clock performance is summarized:

Precision � 40-50 nsec (with 5-10 sample time interval averages,

improving to � 30 nsec with 100 second averaging times)

Accuracy  < 20 nsec  (relative to USNO with multi-hour averages)

SHOWTIME's clock Tsigma error model (described earlier in this document)

performs a real-time estimate of the performance based on my experience

in parametrizing the performance.

----------------------------------

(15) What does the Ionosphere correction do?

The GPS satellites send down an approximate global-average ionospheric

parameter that the user may chose to use; the Motorola ONCORE receiver

permits it to be used, and the [I]ono switch turns it on/off. I have

done tests to check this correction. When the correction was turned OFF,

the 1 PPS epoch time was observed to shift LATE by 18�2 nsec (using the

same 1000-second averaging techniques described above in FAQ #14).

----------------------------------

(16) PRN12 has been mis-behaving. How can I solve the problems it causes?

With Version 2.50, a new "Ignore the bad actors" switch has been added to the

[G]PS Modes screen with the Ignore [G]PS Satellite command; this allows you

to ignore any of the PRN01-31 satellites.

----------------------------------

(17) My "TAC" seems to have locked up. The front-panel LED is on all the time

and nothing seems to work. What do I do?

This glitch has been observed to happen occasionally (most recently to Joe

Taylor at Princeton, who sent his TAC to me for checkout). I have no idea

why it happens, but here is what I did to clear Joe's problem:

(a). I ran his receiver with the Motorola GPS53 controller. It came up

running just fine. Apparently the Motorola software send some binary reset

command I haven't identified in its initialization proceedure.

(b). I also did a "soft" reset on Joe's receiver by disconnecting the 9V

battery. The easiest way to do this on MOST of the TAC's we made is to

unplug the 10-pin ribbon cable from the top board for about a half-hour.

This causes the receiver's BBRAM to get amnesia, and also resets the date/

time clock on the receiver. If you do this, follow the instructions in

FAQ #3 when you start SHOWTIME to set an approximate time into the receiver.

----------------------------------

(18) Sometimes (especially when a really bad set of Lat/Lon coordinates are

in the receiver or when the antenna is in a bad location) SHOWTIME seems to

reset itself a lot, and it then it bounces bad to MessDOS with a nasty,

untelligible error message. Why?

SHOWTIME uses the stock QuickBASIC/DOS "Open COMx" RS232 I/O. It seems that

the PVT-6 receiver can send some garbage characters occasionally and this

caused a lot of problems. In Version 2.60 I spent a lot of effort trapping

the most common of these error and changing the port reset philosophy.

HOPEFULLY the new code cures most of these problems that seemed to occur

only when operating under marginal conditions. Even more work has gone into

solving this problem in the v2.70 release.

In addition to the RS232 I/O problems, there were cases when the NMEA string

parsing routines glitched when satellies were dropping in and out rapidly.

I hope I caught all these cases also.

----------------------------------

(19) Will SHOWTIME and the TAC operate under WINDOZE 3.11 and W95?

In September'95, I got a new 133 MHz Pentium at home, and it came loaded

with WIN95. All the development of SHOWTIME afeter v2.60 was done in an

MSDOS prompt screen in W95. A lot of the testing to try to force glitches

was done by loading W95 with a lot of tasks (including running TASKMON,

NETSCAPE, and playing SOLITARE concurrently with SHOWTIME running in the

background). When I started with the 2.51 code, it crashed all the time in

W95. With v2.60, I seem to be able to run it OK with W95 programs, but it

is unreliable with multiple MSDOS windows open -- presumably because SHOWTIME

has a hard-loop wait for the next 1PPS signal. With v2.70 a number of other

timing-related problems were fixed.

----------------------------------

(20) Occasionally the words "Timing Unreliable" flash on the position/timing

screen. Should I be worried?

The "Timing Unreliable" indicator is computed based on the estimated Timing

sigma, which is based on the number of satellites being tracked. Occasionally

(a) an NMEA message will have bad data or (b) a satellite is dropping in and

out of lock or (c) a new satellite has just been added to the receiver's

tracking list. When these rare events happen, the NMEA message may lag the

event by a second or two, and the calculations get out of step. Don't worry

about an occasional message that flashes briefly; it is only important if it

persists.

----------------------------------

(21) The [&] "speedometer" now (since v2.82) has two numbers. Why?

To better access the internal timing of the program I added a second "stop-

watch" display parameter on the top line (toggled on/off with [&]). The inner

loop of SHOWTIME waits for a new <lf> terminated NMEA "sentence" until XTIME

has elapsed. The first number is the time since the last 1 PPS tick in 10 msec

units, and the second number is a counter of the number of times the software

has passed thru the inner loop waiting for an <lf>.

----------------------------------

(22) I see that I have an "Unlocked for 10 sec" indication, but the "Locked

since" date/time hasn't changed. Why?

The "Unlocked for" counts even brief 1 second dropouts, which may happen

because the receiver re-sorts its satellite list or because there was a

glitch in the RS232 communications. The "Locked Since" is triggered when

the receiver has REALLY been unlocked (i.e. no satellites are detected for

more than LockHyst seconds, the time display is over-written with "LLLLLL"

characters, and then the receiver re-acquires lock). A few "Unlocked for"

counts are not unusual and the counter can be reset to zero by hitting the

[A]veraging [u]nlocked reset keys.

----------------------------------

(23) SHOWTIME ignores the HDOP and VDOP quality flags when I'm running in 2-D

or 3-D positioning mode. Can you put in DOP weighted averaging?

I had resisted this request because for me, SHOWTIME and the TACs were

intended to be used for TIMING tasks, not positioning! But in Ver 2.85

I relented and added the capability. You turn HDOP/VDOP weighting on/off

with the [A]veraging [D]OP Weighting switch (or you can turn it on with

a line that reads "DOPWEIGHT" in your *.GPS file. When turned on, the

position and rms data is weighted proportional to (1/HDOP)^2  for Lat/Lon

and to (1/VDOP)^2 for heights.

The filter length strategy is simiilar to that used before. The positions

and rms differences are still exponentionally averaged as with a weight W:

(Filter Length-W)*(old avg) + W*(new value)

(new avg number) =  -------------------------------------------

Filter Length

with the Filter Length still increasing as PCRATE until it reaches a clamp

value (which you set on the [A]veraging screen). This results in a simple

average when Filter Length < Clamp and an exponential averaging after it

reaches the clamp. What is new is the weight W. In the older SHOWTIME code,

and in v2.85 when DOP weighting is turned off, W is set = 1.

In v2.85 when DOPWEIGHT is ON, W is computed as

W = (1.5/HDOP)^2 for Lat/Lon

and W = (2.0/VDOP)^2 for Heights

The 1.5 and 2.0 are chosen as typical "fairly good" HDOP/VDOP values, so

the W coefficients are normally nearly = 1. Better values of HDOP/VDOP than

these averages will "speed up" the averaging and poorer values will "slow it

down". The filter algorithms are somewhat ad hoc, but I think they achieve

the desired objectives.

-----------------------------------------------------------------------------

POSTSCRIPT:

The SHOWTIME software has been, and continues to evolve -- hence it has

either been blessed by (or suffers from) "creeping featurism". User inputs

are solicited. Send them to me by Email to

clark@tomcat.gsfc.nasa.gov     -or-      w3iwi@amsat.org

Tom Clark                                           February   7, 1995

revised   March     17, 1995

re-revised   April      1, 1995

re-revised   May       23, 1995

re-revised   June       1, 1995

re-revised   June       4, 1995

re-revised   August     1, 1995

re-revised   September 11, 1995

re-revised   September 16, 1995

re-revised   September 27, 1995

re-revised   October   16, 1995

re-revised   October   17, 1995

re-revised   October   24, 1995

re-revised   November   6, 1995

re-revised   November  11, 1995

re-revised   November  14, 1995

re-revised   November  21, 1995

re-revised   December   1, 1995

-=-=-=-=-=-=-=-=-=-= SHOWTIME REVISION HISTORY =-=-=-=-=-=-=-=-=-=-=-=-=-=-=-

Ver 1.x  -- Developmental "Beta" releases.            [Dec'94 to Apr'95]

Ver 2.0 --- The first "public" releases of SHOWTIME.  [April 1, 1995]

Ver 2.10 -- May 23, 1995

- Added the alarm unlock indicator feature.

- Minor bug fixes & display enhancements.

Ver 2.20 -- June 1, 1995

- Improved the unlock alarm functioning from 2.10.

- Added [L] toggle switch to turn the unlock alarm on/off.

- Fixed the "clock sticks in zero-D" bug (due to "feature" in

Motorola's $GPGGA message that doesn't report time in Zero-D).

- Added the [W]GS84 GEOID correction on/off switch ([P]os & [G]PS

screens and GEOID in the *.GPS file) to accommodate Motorola's

new (Version 8.x) firmware changes in the way heights are reported.

- Added  [D]ate/Time Coldstart Clock initialization in [P]os screen.

- Stripped out a lot of spurious code and cleaned up a lot of the

screens to make them more legible and to save code space.

- Improved wording on [SPACE BAR] main help screen menu.

- Deleted the [#] Channel number display (which nobody was using).

- Several menu inputs changed from manual "type a number" entry to

"rotary switches" -- for these, you press the key to cycle thru

the legal values ( [%]ApType, [S]elect DOPtype, [I]on, [W]GS Geoid,

[$]PCrate and [N]oise ticks all run this way now).

- Satellite Status "billboard" rearranged to make room for the new 8-

channel ONCORE display (Untested - I haven't tried an 8-Ball yet!).

Ver 2.21 -- June 2, 1995

- Corrected a minor bug in reported heights when running in

positioning mode with old (pre-V8.0) ONCORE code. The [W]GS84

switch in the [P]osition screen now reads "new" vs. "old" to

correspond with pre- and post- Version 8.0 receiver firmware.

- Entering a [M]anual position in the [P]osition screen now

"seeds" the average position with the new reference position.

- In the [M]anual [H]eight entry, you can now enter either WGS84

or MSL heights and SHOWTIME will do the conversion for you,

providing the [W]GS GEOID switch is in the correct position for

your receiver's firmware. The correction is based on the WGS84

minus MSL Geoid separation computed in the Motorola receiver (the

value of which is reported to you on the [P]osition screen).

Ver 2.22 -- June 4, 1995

- Cleaned up & added some more logging info to the *.GPS file.

The logging data in the file now includes the number of seconds

included in the Lat/Lon/Hgt/Tsigma averages, the "Valid Since"

last locked time, and the number of times it was unlocked. Note

that these parameters reflect the last time you write the *.GPS

file with a [F]ile [U]pdate.

- The [P]osition screen now shows the RMS of (REF minus AVG)

- When you are in zero-D timing mode, the reported Average position

is set to the 3-D REFerence position. In 2-D mode, the Hgt is set

to the REFerence Hgt.

- Removed the [F]ile [K]ill *.GP$ command (not needed with [S]hell)

Ver 2.23 -- June 19, 1995

- Made a few minor changes to make software recover from temporary

data outage on RS232 cable better.

- Small change to screen that tells when PC clock has been [S]et.

Ver 2.30 -- August 1, 1995

- The Lock Alarm logic that was added in Ver 2.10 had problems!

A new, more robust logic was added in 2.30. The problem occurred

because of the way that I counted the number of satellites that

were in lock using the GSA message. The new logic uses the GGA

report directly from the receiver. Showtime now has an added

parameter (LOCKHYST in the *.GPS file, which can be set manually

on the [G]PS Mode Screen) to act as a hysteresis "damper" to

supress brief dropouts. A value � 2-3 seconds for the hysteresis

seems to work well.

Ver 2.40 -- Sept. 11, 1995

- Corrected minor bug in entering cable lengths -- Meters units

didn't work correctly, and the units code got confused if the

units had an "S" in them (i.e. MSEC).

- The Time Offset screen was made a bit more explanatory. Several

other cosmetic screen changes also were made.

- The Timing Offset "mini-screen" in the upper right-hand corner

(toggled on/off with the [=] key) was streamlined and (hopefully)

made more explantory. This screen is now framed in a box.

- The (ham) Maidenhead Grid Square computation was streamlined.

- Change to RS232 COM1/2 port initialization seems to improve glitch

recovery reliability.

Ver 2.50 -- Sept. 16, 1995

- Added the ability to allow the user to disable & ignore multiple

GPS satellites. In the [G]PS Screen, the [G]PS Use/Ignore key

allows you to toggle any of the satellites PRN01-31. (You cannot

toggle PRN32 on/off. This is because the ON/OFF status is sent to

the Motorola as a 32-bit unsigned array, one bit/satellite. Alas,

QB45 does not have an unsigned long integer type, and it would have

been a huge pain to add the 32nd bit to the field. Sorry!). This

was added because the antique Block-1 PRN12 satellite has been

unreliable. In the [G]PS modes screen, good satellites are shown

as "+" and bad ones are flagged "�". The "�" character also is

shown next to ignored satellite's PRN in the satellite status

window in the lower right-hand quadrant of the screen.

- A new parameter "IGNORE" appears in the *.GPS control file to save

the status of the new parameter as a 31-character string where

a "+" means that the satellite is to be used an any other character

(like a "-" or "�" or a blank) means that the PRN is to be ignored.

- To make room for the Ignore feature, the ability to exit SHOWTIME

with any of the 7 "legal" NMEA messages being set to user-defined

rates ("EXIT NMEA") has been discarded. "EXIT BINARY" still works.

- In general, a lot of cleanup was done on various screens to make

the SHOWTIME code more compact. Also, several small, related

subroutines were merged and/or put into inline code to make the

program a bit smaller and faster.

- The display that shows the testing of the 1PPS RS232 tick at

startup has been changed from ....#..#.... to something that looks

more like an oscilloscope trace.

- The calculation of Local/Greenwich Mean Sidereal Time (LMST/GMST)

was streamlined. Earlier versions included the quadratic (but not

cubic) term to calculate the GMST at 00:00 UTC based on elapsed

time from the J2000 epoch. For computational efficiency, the small

quadratic term is now "locked" to 1996.0. The error for the next

several years is < 1 msec.

Ver 2.51 -- Sept. 27, 1995

- In 2.40 & 2.50 I streamlined the RS232 configuration. Unfortunately,

it didn't work on all machines. Some RS232 ports default to having

the CTS and RTS lines low and some default to high. The 2.51

software has been modified to IGNORE the unused handshaking lines.

(thanks to the folks at Onsala for giving me an opportunity to

duplicate this problem).

Ver 2.60 -- Oct. 15, 1995

- A lot of work was expended to better trap RS232 I/O problems. The

PC's 16450/16550 UART is now issued a hard reset each time the I/O

is initialized. Error trapping has been improved. Several variables

are now tested to make sure they don't go beyond reasonable bounds

if the PVT-6 receiver send some garbage data.

- The DCDmask variable was hard-coded instead of being set in the

*.GPS file, so users MUST input the 1PPS signal on the DCD line.

Ver 2.61 -- Oct. 17, 1995 (minor maintenance fixes)

- Still had to trap a couple of more bad data instances. Minor update

from Ver. 2.60

Ver 2.70 -- Oct. 22, 1995

- Did a lot of testing to force errors that could cause SHOWTIME to die

and implemented a LOT of new traps. All these involve possible bogus

data returned in NMEA messages if the computer loses sync with the

RS232 NMEA messages while running in W95.

- To make space for all the new traps, a number of separate subroutines

were made into in-line code or subroutes in the MAIN module. This

should also speed execution a bit.

- A few minor cosmetic changes were made to the display,

Ver 2.71 -- Oct. 24, 1995 (minor maintenance fixes)

- Found another sanity test that was needed.

- Cleaned up and slightly revised the [F]iles screen logic

Ver 2.72 -- Nov. 4, 1995 (never released publicly)

- Fixed minor bug in the display of the nsec portion of the [U]+[I]

total time offset in the [T]iming screen and the [=] display window.

Calculations were done correctly -- the display was wrong because

an integer number was displayed in a floating point field.

- The PCrate switch somehow had gotten broken a few versions ago.

It was made to operate again and the logic to set the GGA/GSA/GSV

NMEA messages at a slower rate (PCrate seconds) was revamped to

be more reliable at both 2 & 3 second "slow" rates. The new logic

may glitch if you try to start SHOWTIME withing about 10 seconds

of the PC's Midnite marker.

- The [$] PCrate switch is now available on the [A]veraging screen

in addition to the [G]PS Receiver Mode screen. On either screen,

hitting [$] increments PCrate in a [1,2,3,1,2,3...] sequence.

After hitting [$] on the [A]veraging screen, the entry changes to

the ColorRED color and SHOWTIME resets to initialize the new rate

on exiting with <esc>.

Ver 2.80 -- Nov. 6, 1995

- Made a number of changes in the logic behind the Position/Timing

window so that the top line more correctly reports the receiver's

status and that the unlock detection is more reliable.

- Additional "sanity" checks were added to prevent bogus coordinates

from being included in the position (especially heights).

- Cleaned up & tested the new widgets added in 2.72 for public release.

Ver 2.81 -- Nov. 8, 1995

- A minor bug caused an error in [F]ile [L]oad -- a new *.GPS file was

not loaded correctly. A global variable got used for two different

purposes during one of the recent changes.

Ver 2.82 -- Nov. 11, 1995 (minor maintenance fixes)

- The earlier versions of SHOWTIME would report incorrect Lat/Lon

if either Lat or Lon was between 00 and -01 degrees. This is a

common problem with a lot of software since it is hard to represent

a signed negative zero integer!  This minor problem is now fixed.

- The -00 problem was not trivial to fix. Added a new parsing function

(Pd$) to make it possible. This resulted in a slight display change,

namely that Longitudes under 100 degrees (+ or -) now have the

degrees field zero-padded to 3 characters plus sign.

- I also made it possible to [P]osition [M]anual enter Lat/Lon in the

-00 to -01 degree band.

- Fixed a minor bug that somehow crept in that made the [C]olor and

[A]veraging screens not work right when in zero-D timing mode.

- The [&] "speedometer" has a second number displayed -- this is the

number of times thru the inner loop trying to read an NMEA message

during the current second.

- A screen which shows the "Locked Since" and # Unlocked information

appears when a non-operative menu key is struck. <esc> clears this

information panel and resumes operation. (The same info also appears

when you manually [S]et the PC's clock.)

Ver 2.83 -- Nov. 14, 1995 (minor maintenance fixes)

- Found a minor glitch (which crept in from earlier changes) when

running in forced 2-D mode with SHOWTIME not recognizing the

reported rcvr state properly.

- Fixed glitch where [A]veraging [z]ero didn't reset all the reported

statistical parameters -- some variables had been left out of COMMON.

Ver 2.84 -- Nov 21, 1995 (minor maintenance fixes)

- Fixed minor bug that caused program to lock up if you tried to

[F]ile [L]oad a non-existent file -- an IF test was wrong.

- It was possible for the [P]osition or [R]MS or [T]iming Sigma filter

lengths to not "clamp" at the proper values if you decreased the

numbers and they were already greater than the new filter lengths.

- The Pos/RMS filter weights had been a mish-mash of floating and

integer values. They are now uniformly long integers and a single

subroutine handles incrementing them each time new data arrives.

- There had been some confusion on the Position screen about �Avg &

�Ref. The nomenclature was changed to �(RA) (meaning the difference

Ref-Avg), �(AL) (meaning Avg-Last) and �(RL) (meaning Ref-Last).

- Some essentially useless (and probably buggy) code that attempted

to sort the "S-meter" display panel when two satellites are at the

same elevation to have the rising satellites listed above the setting

satellite was deleted. The ordering for this rare case is now only

determined by the order the satellites are reported in the NMEA msg.

Ver 2.85 -- Dec 1,1995

- In response to a request, the 2-D or 3-D positions can now be

averaged based on (1/HDOP)^2  [Lat/Lon]  or  (1/VDOP)^2  [Hgt].

A new switch [A]veraging [D]OP Weight has been added -- pressing

the [D] key will toggle the xDOP weighting on/off. In the normal

Position window, you can see when the weighting is turned on

by a letter "w" appearing before the positions differences and

rms indicators.

- A new switch has been added to the *.GPS file called DOPWEIGHT.

If the line is present, then the xDOP weighting is to be applied,

if it is absent, then the weighting is not used.

- To squeeze the [D]OP Weight item into the [A]veraging screen, the

[A]veraging screen layout was changed.

- To accomodate this change, the weighting filters inside SHOWTIME

were changed from LONG (integer) to DOUBLE (floating).

- The filter length strategy is similar to that used before. See

FAQ #23 for more details.

Ver 3.00 -- Jan 7, 1996

- MAJOR update -- changes are documented in the separate file

named SHOWTM30.DOC.

######