STREAM protocol

---------------

This is a high speed binary transfer protocol.  No acks are made.

Packet format:

--------------

If the high order two bits in the length are both one then the block

is an information block.  CRC are 16-bit cyclic-redundancy-check using

CCITT standard.  All bytes are included,

End-of-file

Length       -- 2 bytes     ** both are ZERO **

Block number -- 1 byte

Normal

Length       -- 2 bytes     ** Max 49151

Block number -- 1 byte

Data         -- variable

CRC          -- 2 bytes

Information block

Block type   -- 1 byte      Range $C0 - $FF

Length       -- 1 byte

Block number -- 1 byte

Data         -- variable

CRC          -- 2 bytes

Field descriptors:

------------------

Length -- Length of the block including all headers/trailers.  The

exception to this is the EOF marker

Block number -- Incremented block number for all blocks starting with ZERO.

After 255 wrap back to zero.  This is in all blocks including the

EOF marker

Data   -- Any data.

CRC    -- CRC are 16-bit cyclic-redundancy-check using CCITT standard.

All bytes are included,

All information is transmitted in NETWORK BYTE ORDER.  High order byte

is first.  This is the REVERSE of the INTEL 8088 chip.

Data area for Information Block

-------------------------------

File info -- High order byte of packet is $F0

Data area is formatted as one or more values

type   -- 1 byte  -- value type

length -- 1 byte  -- length of value

value  -- n bytes -- actual value

Value types

0 -- Length of file -- 4 bytes long