**********   Operating instructions   **********

* A '#' in the following doc,means an argument number.

*  The following commands are available  from  the  SYSOP's

window :

ATT X Y   : Attaches a com port specified by  X  with  baud

rate Y. X values are : 0 for com1,1 for com2

etc.  Y values are : 1 for 1200 bauds,2 2400,3

4800,4 9600.Press Alt-T to abort this procedure

instead  of ESC.

---------

C MSG#    : List of fwd stations are going to rcv this msg#

CL        : List of fwd stations,followed by the total number of msgs

remain to fwd.

---------

DU        : Display users.

DU XXXXXX : Display user XXXXXX.

DS        : Display sysops.

DR        : Display registered users.

DX        : Display expert users.

---------

EXAM PATH\FILENAME.EXTENSION  :  Access  to  the  built  in

editor,by editing the above example file. The max length of

the file you can edit,depends on the amount of free memory.

Basic Editor commands :

F2  : save file.

F10 : return to sysop window without save.

^K^B : mark block start point.

^K^K : mark block end point.

^K^C : copy block on cursor position.

^K^H : release block.

^K^Y : erase block.

^Q^F : search forward.

^Q^A : replace a set of characters with another set.

^O^N : Goto line N.

Home,End,Arrows for cursor movements.

Pgup,Pgdn : change window up,down.

----------

EM #    : Edit header of msg #.

EU call : Edit user header.

----------

F        : Start forward.

F xxxxxx : Forward to xxxxxx station.

FR xxxxx : Reverse fwd only to xxxxx station.

FWDSTOP  : Stop fwd at once.

FWDX     : Stop fwd to the  current  station  and  continue

with next.

FM x     : Change fwd minute to x.

FT x     : Change channel timeout to x minutes.

----------

GF FILENAME.EXTENSION : Change forward file.

----------

H   : A list of cmd mnemonics.

----------

I   : Displays info file.

----------

J   : Last connected stations.

J # : Stations heard on port #.

----------

K #     : Kill message which number is #.

KM      : Kill messages addressed to my call.

KF      : Kill forwarded messages only.

KY      : Kill all msgs with 'Y' status.(Does  not  include

bulletins)

KP      : Kill all private msgs.(Use with care)

KH      : Kill all held messages.

KT      : Kill all traffic msgs.(Use with care)

KB      : Kill all bulletins.(Use with care)

K> xxxx : Kill all msgs addressed to xxxx.

K< xxxx : Kill all msgs come from xxxx.

K@ xxxx : Kill all msgs with xxxx @field.

KU xxxx : Kill user xxxxx.

----------

L,LN   : List new messages.

L #    : List until message number #.

LL     : List Last message.

LL #   : List Last # messages.

LK     : List killed messages.

LF     : List forwarded messages.

LH     : List Held messages.

LM     : List messages addressed to my call.

LP     : List all private msgs.

LB     : List all bulletins.

LT     : List all traffic msgs.

LE     : List all extended msgs.

LY     : List all msgs with 'Y' status.

LU     : List unread messages addressed to my call.

L> xxx : List messages addressed to xxxx.

L< xxx : List messages originated from xxxx.

L@ xxx : List all messages which have @xxxx.

L* xxx : List all msgs which have the word xxx in their subject field.

L> xx* : List all msgs which have the 'xx' substring in their to address.

L< xx* : List all msgs which have the 'xx' substring in their from address.

L@ xx* : List all msgs which have the 'xx' substring in their @BBS address.

L x y  : List msgs included between x(major),y(minor).

----------

MF # PATH\FILENAME.EXTENSION : Make file from message which

number is # to the above path,with the above filename.

MAKE : Starts housekeeping,by  reading  make.bbs  file  and

following the instructions which includes.You  can  add  or

delete instructions by editing this file. This procedure is

also  automatically  controlled  from   the   configuration

file.BBS,executes  make  instructions   once   during   the

predefined days.You  can modify days,hours and  minutes  in

the cnf file.Blank or erase this file if u  don't  want  to

execute make. If make doesn't work,this means that there is

a connected channel. There is an  alternative  way  also,to

avoid erasing and rewriting lines in make file.Just put  an

'*' in front of these cmds you don't want to execute.

* Make instructions:

It's important to write these instructions in capitals.

KILL  KILLED       : Clears  the  data  base  from  killed

messages.

F xxxxx            : Optimizes xxxxxx fwd file.You can have

more than one.

OPT JFILE          : Optimizes jlist (connected) file.

OPT USERFILE DAYS  : Optimizes userfile.

EXP ADDRESS DAYS   : Automatic  kill  of  msgs to a given

address,after the desired number of

days we have receive them.

EXP *** DAYS       : Automatic kill of any msg,after the

desired number of days we have

receive it.

OPT BIDFILE DAYS   : Automatic erasing of expired bids,after

the desired number of days we have

receive them.

RENUMBER           : Automatic renumbering of msgs.Starting

value will be #200.

Example of a possible make file:

OPT BIDS 120      (Kill these bids which are 4 months  (120

days) old.)

OPT JFILE

OPT USERFILE

EXP ALL 20

EXP AMSAT 50

EXP SYSOP 10

EXP *** 120

KILL  KILLED          (It  is  better  to  put   it   after

expire,for best performance)

F SV1IW

*F IT9UBA           (BBS ignores this line)

F SK3AH             (You must always put 'F' commands last)

----------

OUT x        : Force disconnect on channel 'x'.

----------

R #          : Read message which number is #.

RV #         : Verbose read of msg #.

RM           : Read all messages addressed to my call.

RF PATH\FILENAME.EXTENSION : Read a  file  from  the  files

area.

REPLY <text> : Send  specified  text  on  all  connected

channels.

RT           : Enters the round  table  mode.If  there  are

users in the table,sysop has its own  vision

from its own window.Press ALT-T to leave

the round table.

----------

S,SP,SB,ST,SS,SE :Send  a  message  (message  type  may  be

P-private,B-bulletin, T-traffic,S-same,

E-extended)  to address.If  we   send   a

private msg to a valid call,it will  go

to  one  station  only,even  if  we  have

declared more destinations in the dest.fwd

file.The only way to send it to all of  the

predeclared destinations is the 'S' type.

BBs fwds this msg as a private  msg to

all destinations.

Syntax :

SP SV7AIZ, OR

S  SV7AIZ, OR

S  SV7AIZ  @ SV7AIZ,OR

SP SV7AIZ  @ SV7AIZ,OR

SB XXXXXX  @ XXXXXX < XXXXXX $XXXXXXXX

|    |         |        |       |

|    |         |        |       |_ Bid

|    |         |        |_From

|    |         |_Atbbs

|    |_To station

|_Type

There is an automatic BID generation option.BBS adds a  BID

when we type the following:

SX XXXXXX @ XXXXXX < XXXXXX $

So,a standalone '$' cause bbs to generate a BID.

-----------

TTM PATH\FILENAME.EXTENSION : Text to mail from  the  above

file. Do  not  use  this  procedure   when   a   connection

occurs,unless the text file is short.Because it  locks  the

bbs,for full speed.

-----------

V     : BBS version.

------------

W     : Contents in files dir.

W xxxx: Contents in subdir xxxx.

Z PATH\FILENAME.EXTENSION : Erase the above file from files

area.

------------

YW    : Contents in binary files area.

YZ PATH\FILENAME.EXTENSION : Erase the above file from  bin

area.

------------

#     : Sysop available-not available toggle.

------------

Available global keystrokes :

F1-F9          : Logical channels 1-10.

@F1-@F10       : Logical channels 11-20.

Ctrl:F1-CtrlF4 : Logical channels 21-24.

F10            : Monitor channel.

Home           : Sysop window.

Pgup,dn        : Scroll monitor window.

Uparrow,Dnarrow: Scroll channel window.

Larrow,Rarrow  : Keyboard buffer back-ahead.

INS            : General info(Maxkld,Maxheld,maxfdd,Memory,channels,etc).

Alt-E          : Built in Calculator.

Alt-S          : Screen save.

Alt-X          : Program termination.

Alt-T          : Terminal-BBS toggle on all channels,except

monitor.  If  no  activity  from  our side

in   10 minutes,while we are in terminal

mode,  then   it   goes   to   bbs   mode

automatically.

Ctrl-Home      : Dos task:

BBS processing doesn't stop.

(BBS processing stops if you  run  another

program.

Starts again when this program deallocates

memory.)

Press a channel key or Home key  to  leave

dos task.

Ctrl-Pgup       : Toggle.It has  to  do  with  the  monitor

window,while we are using the editor.If

it is  on,then we can see monitor frames

while editing a file.

Alt-P           :  It  works  only  in  terminal  mode  and

toggles between private and normal mode

conversation.

Alt-D           :  Download  a  file  manualy  in  terminal

mode.^

Alt-U            :  Upload  a  file  manualy  in   terminal

mode.Close the file by pressing Alt-C.^

Alt-B           : Download a binary file in terminal mode.^

Alt-R           : Upload a binary file in terminal mode.^

^ . You have to use manual up-dnloads in terminal mode with

care,because there are no error responses.They have nothing

to  do   with   the   standard   predefined   files,bin,etc

directories.You must give full paths.

**********   REMOTE  SYSOP  **********

@          : This  command  is  available  only  on  remote

sysops.It has no effect on  sysop  window.If u

wish  to  have remote access to your bbs,then

you have to send  in    a channel  a '@'.  BBS

responds    with    some    numbers.    (Max 5

numbers).These numbers have to  do  with  your

password,which  you  have declared in  the cnf

file.You must send one letter of  your

password,for each number.

For example:

Our password is SPiRoS.

We send a @.

Bbs responds :

Remote Sysop Verification : 1 3 4 2

We must send : SiRP,to have remote access.

Bbs responds : REMOTE >  (ok)

Else Bbs responds  :You  are  not  a  Remote

Sysop.

-  Password's  length  must   be   up   to   25

characters.

-  You have to mark yourself,or any  other,as  a

sysop in the user's data base,if u  wish  to

have  remote access.Otherwise, bbs responds

with 'invalid command.'.

-  Another '@' cause bbs  to  return  to normal

mode operation.

REMOTE CMDS  : All sysop-window  cmds  are  available  with

some minor changes:

In case of DU,DB,DS,DX,DR:

PU    : Display users.

PB    : Display bbss.

PS    : Display sysops.

PX    : Display expert users.

PR    : Display registered users.

PE    : Display excluded users.

Cmds like U,D,W,Z,have also a different way  of  use.If  we

add an '*' in  front  of  each  path,bbs  ignores  all  the

predefined paths in the cnf file and follows the exact path

we have typed.

For example:

If we have declared our files directory, in the  cnf  file

as FILES,and we send the cmd 'W',bbs searches the directory

FILES. If we  send  a  'W  *\',bbs  will  search  our  root

directory.

More examples follow to get the difference:

Assuming that we have defined C:\BBS\FILES\  dir  in  the

cnf file,like the above:

W                 =    DIR C:\BBS\FILES\

W ALL             =    DIR C:\BBS\FILES\ALL\

W *\              =    DIR C:\

W *\TOOLS\        =    DIR C:\TOOLS\

W *D:\            =    DIR D:\

U DX\DX.BTN       =    UPLOAD TO C:\BBS\FILES\DX\......

U *\DX\DX.BTN     =    UPLOAD TO C:\DX\...

D DX\DX.BTN       =    DNLOAD TO C:\BBS\FILES\DX\......

D *\DX\DX.BTN     =    DNLOAD TO C:\DX\...

Z DX\DX.BTN       =    ERASE C:\BBS\FILES\DX\......

Z *\DX\DX.BTN     =    ERASE C:\DX\...

Plus,

#     : System's reboot.

!     : Make procedure.

- X    : Disconnect channel <X>.

Note*.There is no chance to change fwd file  during  remote

operation.

------------

**********  Wildcards :  **********

$D   : Date in header format.

$d   : Date in full description.

$T   : Time in header format.

$t   : Time in full description.

$U   : Usercall.

$N   : Username.

$F   : Fwd minute.

$E   : Current channel.

$C   : Current msg in header format.

$V   : Our call.

$S   : Our name.

$L   : Last msg number.

$A   : Number of active msgs.

$y   : User's last login date.

$x   : User's last login time.

$z   : User's Zip code.

$h   : User's Home BBS.

$o   : Total number of current connections.

$O   : From address field in header format.

$b   : @bbs field in header format.

$B   : BID field in header format.

$s   : To address field in header format.

$v   : S/W version in header format.

--- Notes for users. ---

This is what we see when issue an EU cmd:

Call     Name        TBL  Ct   Zip   LastLogin      Home

|         |          |||  |     |       |            |

|         |          |||  |     |       |            |

|         |          |||  |   Zip code  |            |

|         |          |||  |             |            |

|         |          |||  |              |             Home

bbs.

|         |          |||  |            No explain

|         |          |||  Login count

|         |          ||Bidlevel

|         |          |Bidok

|         |         usrtype

|         Username

User call

1.There are 6 user  types.(U)nregistered  user,(R)egistered

user,(B)bs,(X)pert user,(L)ocal user,(S)ysop.    Each  user

has also a bid flag(BidOk).If this flag  is  'N'  then  bbs

rejects   these msgs which include  bids,coming  from  this

user.Its default status    is  'Y'.You  have  to  fix  that

manualy by using the 'EU' command.  You have  to  mark  fwd

stations as bbss by using the 'EU' command.

If your fwd path with  a  station  is  only  for  private

mails,then, you  better have to avoid bids.(Bid flag ='N').

All  other paths must  have  bid  access.(Bid  flag  ='Y').

(Don't  forget  to  put  the  right  value   on   the   bid

flag,because u'll never   take fwd).

2.Most of  the  new  multiuser  bbss  use  additional  bid

responses,to avoid  bid duplications,fwd  delays  etc.These

new responses are:

LATER

>

Which  means,send  this  bulletin  later  for   the   final

ack,because someone else is trying to send  it  on  another

channel.

REJECT

>

Which means,something is wrong from the sender's side.

Well,bbs has to know these kinds of  stations,so  another

usertype field has been added which has to do with the  bid

level (Bidlevel). If you have fwd path with a station which

contains this level,during fwd synchronization,  bbs  knows

automatically what to do.In all  other  cases,you  have  to

mark  him.

----------

Enjoy,Spiros.