posted by KC3ET @ KA3NVP

TRANSLATION BY ROBERT BARRY

SEPTEMBER 28, 1989

PROTOCOL FOR COMMUNICATION ON THE RS232 CUT POINT

1. Host - Terminal Interlink distinctions

Because and RS232 cut point can act as a node and as a terminal,

some means for distinguishing the two modes must be provided.

This is done simply at the input to the DCD of the SIO channel B.

Normally, the RS232 cut point is in the terminal mode. Following

instructions given in the hand book, a negative potential is

provided to the DCD connection, and Interlink mode is indicated

by this.

2. Frame Format in Interlink Mode

All frames are sent as on an HDLC channel. The software makes

no adjustments at Level-2 and higher. In Level-1, just the

HDLC flag needs to be reset. Thereby, a simple variant of the

HDLC protocol is attained.

All frames begin with STX = 0x02 and end with ETX = 0x03.

Inside a frame STX is replaced DLE-STX and ETX is

replaced by DLE-ETX. DLE (0x10) is extended to DLE-DLE.

For fault detection, the common CRC is not used. Since

faults on an RS232 link are not very likely, a simple check sum is

appended to the ETX. The check sum is 8 bits long and applies to

all the information in the frame (without STX-ETX). The trailing

DLE's are not included.

3. Collision Avoidance.

With only two TNC's on an RS232, no protocol, obviously, is

needed. All transmittals run quasi-full duplex. With more than

two TNC's, a CSMA-CD driver (or controller) is used, which is

controlled over RTS-CTS. Each TNC may only begin a send-cycle if

it has through the CTS determined that the bus is free. It

asserts RTS and loads the bus. Then a randomly determined time

(from 2.5 to 25 milliseconds) is allowed to pass so that any

other TNC's who are ready to send can recognize the change in

channel status. If, during this wait time, some other TNC asserts

RTS, then the send-cycle is broken.

THE INTERLINK PROTOCOL

0. Foreword

The nodes exchange a special protocol for information display.

The frames of the protocol have a 0xCF as Level-2 protocol

identifier. Each frame contains a special Transport Header after

this Level-2 Header (AX.25), which consists of 20 bytes. Because

the AX.25 protocol permits only 256 bytes of information, overly

long User Frames are divided into two sections. TheNet takes care

of this, so that the original size is retained. To keep best use

of the network, no frames longer than 236 bytes are allowed

(Mailboxes). Please note that the Level-2 frames used between two

nodes have the call signs of both parties included (because there

is no digipeater list), whereas in the Transport Header, the call

sign of the level 4 partner appears.

Examples: DB0FD contains a frame for DB0KH from DB0FC, which must

be sent to DB0FE, because DB0FE is the next neighbor on the path

list. DBFD, then, appears in the Level 2 Header as the Sender,

and DB0FE appears as the receiver. In the transport header,

DB0FC is the Sender and DB0KH is the Receiver.

1. Construction of the Transport Headers

The first 15 bytes are evaluated by all parties to find the

optimum path. The remaining five bytes take care of failures

between sender and receiver. (AX.25 is by definition failure

free.) The error provision is necessary, because at Level 2, the

system cannot know if the entire path exists or not. In case of

sudden loss of path, the Receiver must have some means of sending

back frames to the sender for reclaiming.

1.1 Originating Node's Call Sign

Length, 7 bytes, normal AX.25 (packed bytes) format

1.2 Target Node's Call Sign

Length, 7 bytes, normal AX.25 (packed bytes) format, EOA bit set.

1.3 Remaining Packet Life Time

The Remaining Packet Life Time is initially set by the

originating node, and it is reduced by one by each transporting

node. When the value reaches zero, the packet is destroyed. No

error message results. This leads to one-way-paths. For example:

DB0FC initializes its packet to a remaining life of 64, but DF6LN

uses 10. The path between DB0FC and DF6LN has 15 intermediate

nodes. A packet from DB0FC arrives with a remaining life of 49,

but the reverse message is destroyed in passage because 15 > 10.

A life time limit is necessary to forestall perpetual loops in

the system.

1.4 Four bytes with meaning depending upon each Opcode

These bytes are called B1, B2, B3, B4, in the order they appear in

the frame. In each virtual channel, two bytes are used by the

sender and receiver for identification, at the time that the

channel is constructed at Level-4. One of the bytes of the index

is the running count in the circuit list. The other, ID, byte, is

a random number, to prevent ghost frames from a prior message

from circuilating in the system.

1.5 Flags and Opcodes (1 Byte)

The lower four bits give the Opcode. The upper four bits are

flags.

1.5.1 Opcodes

Presently, only six of the possible 16 Opcodes are defined.

1.5.1.1 Opcode = 1  Connect Request

Created for a Level-4 Connection.

B1 = My Index

B2 = My Id

B3 = not defined

B4 = not defined

The frame has the following information:

Proposed Window Size, Level-4 (1 Byte)

For better utilization of the system at Level-4, to

permit more frames to be interjected. This costs RAM. Each Sysop

on the system must make his own choice.

Call sign of the End User. (7 Bytes, AX.25 format with

packed bytes)

Call sign of the Sender Node, (7 Bytes, AX.25 format with

packed bytes)

1.5.1.2 Opcodes = 2  Connect Acknowledge

Answer to a connect request. The choke bit shows, in this case,

that no free places exist on the free list, and the connect

request is declined.

The information in the frame is:

Unified (or agreed) window size, Level 4 (1 Byte)

The unified (or agree) window size is never greater than

the proposed window size.

1.5.1.3 Opcode = 3  Disconnect Request

A directive to relese the Level-4 connection. The order can

come from either end of the connection. The Receiver is ordered

to send along any intermediate information and then to disconnect.

B1 = Your Index

B2 = Your Id

B3 = not defined

B4 = not defined

The frame contains no information.

1.5.1.4 Opcode = 4  Disconnect Acknowledge

Assers that a Level-4 connection has been cut.

B1 = Your Index

B2 = Your Id

B3 = not defined

B4 = not defined

1.5.1.5 Opcode = 5  Information

In these frames, information is translated.

B1 = Your Index

B2 = Your Id

B3 = Running number "send sequence"

B4 = Running number "receive sequence"

Each frame is given a B3 number as it is sent. This serves as

the value for the receiving frame of the same Level-4

connection to (B4 - 1). This value corresponds to the Level-2

value with the change that the "send sequence" does not run from

0..7 but rather from 0..255.

1.5.1.6 Opcode = 6  Information Acknowledge

While the status of the contained information is implicit in the

Info Frame, in one-sided data flow, this item is still necessary,

acting as an extra confirmation.

B1 = Your Index

B2 = Your Id

B3 = not defined

B4 = Running "receive sequence" number

This frame contain no information.

1.5.1.7 Not Defined Opcodes

All other Opcodes indicate that the frame has been destroyed by

the Receiver.

1.5.2 Flags

The upper four bits in the Opcode Byte are reserved for flags.

1.5.2.1 Bit-7  Choke Flag

The node does not have enough memory free to take in more

information for the present Level-4 connection.

1.5.2.2 Bit-6  NAK Flag

Contrary to what was said above, the Receive Sequence is not used

as confirmation but as a directive that the Frame having this

number is to be retained.

1.5.2.3 Bit-5  More Follows Flag

The current frame must be divided. All following frames

including the first one not having this flag are to be

concatenated into one by the receiver.

1.5.2.4 Bit-4  Not Defined

END

(de:DF2AU)