Performance Monitoring

-or-

"I wanna fix it, is it broke?"

Skip Hansen, WB6YMH

Harold Price, NK6K

Presented at the 6th ARRL Computer Networking Conference

Redondo Beach, California, August 1987

Abstract

Much of the performance information on Amateur Packet Radio is

anecdotal and ephemeral; a subjective and non-detailed account

usually limited to a gross statement of "goodness" or "badness",

which is neither well documented nor long remembered.  While

there are several papers which describe the expected performance

of CSMA-type systems, there is little actual data about the live

amateur packet system.

The authors discuss the need for accumulating performance data

and describe work in progress to supply performance measurement

software using a C program and a TNC with KISS software.

1.  Why Performance Monitoring?

Big changes are coming in amateur packet radio.  In early 1987,

most of the amateur packet network was based exclusively on AX.25

and digipeaters.  By the end of 1988, if not sooner, much of the

packet world will be made of up a conglomeration of NET/ROM,

TEXNET, TCP/IP, and other systems interconnecting 40,000 AX.25-

based users.  Each will be implemented and installed by

packeteers eager to make the network better than it was before.

Each system contains a myriad of trade-offs and compromises.

Each system has several tuning knobs which can be used to

modify the way it operates, affecting both local user

performance and global network performance.  In many cases, these

knobs will be cranked by people with no data on how things are

running and therefore no way to tell if anything got better.  In

other cases, the knobs will be tuned to optimize local

performance, to the undetected detriment of the rest of the

network.

Performance data is vital to a local network.  It is needed

before the current network can be tuned, and it should be

available to those who will help specify the next network.  Put

simply, if you don't know what you have now, how will you know

if what you get next is any better?

1.1  We've already missed one chance.

We've already missed one chance to monitor a major change, and in

California, we've missed a second.  The first version of AX.25

did not use the Poll/Final facility of LAPB.  In that version, if

an acknowledgment of a data frame was not received, the data

frame was re-transmitted.  If multiple data frames were

outstanding, only the first one was re-sent. In the second

version of AX.25, the poll/final facility was implemented.  In

AX25v2, if a data frame is not acknowledged, a "poll" is sent

out, soliciting a new acknowledgment.  If that ack does not

indicate that the data frame was received, the data frame is then

retransmitted, otherwise transmission continues with new data

frames.

Any change to a protocol like the one described above entails

some cost.  Whether it is the effort involved in updating and

distributing new software, or the trek to a snowed in

mountaintop to swap ROMS, some of our limited people resources are

expended.  In the poll/final update, was a improvement in network

performance obtained that in some way offset the effort involved in

implementing it and updating the user base?

Unfortunately, we'll never know.  Since there was no network

performance data before the change, and none was taken after,

there is no way to tell.  Our only indication is indirect; one

of the original major proponents of the change to poll/final is

now suggesting that poll/final not be used in some cases. [1]

For future changes, we must do better.

1.2 NET/ROM

In California, the old digipeater backbone which connected

northern California, southern California, and Arizona has been

largely supplanted by NET/ROM nodes.  We had no data showing the

performance of the old system, and we have no data on the

performance of the new system.  It is therefore difficult to

measure the improvement.

2.  Field Experience vs. Theoretical Predications.

There is a large amount of literature on the topic of packet

switching systems, and on packet radio.  Some is quite accessible

to the average amateur, one networking textbook in particular, by

Tanenbaum [2],  has been cited so often that it is stocked by

local amateur radio stores.  There is little written, however, on

packet as it is practiced in the amateur radio world.  In most

cases, if you notice the discussion leaning toward the way we do

it, you find it given as an example of the wrong way.  Actually,

the word "wrong" is seldom used, "less optimal" is more common.

Much of the non-amateur networking experience of those who make

up the amateur packet radio community is in the area of local

area networks (LANs).  Although there are a great many common

problems and solutions between commercial LANs and the amateur

packet network, there is a danger in assuming calculated

performance parameters for the former have relevance in the

later.  Unfortunately, there is a tendency, with a lack of actual

data, to use predicted LAN data in design and implementation

discussions as if it were gospel.

A LAN, as discussed in Tanenbaum [2] page 286, generally has three

distinctive characteristics:

1.   A diameter of not more than a few kilometers.

2.   A total data rate exceeding 1 Mbps.

3.   Ownership by a single organization.

Although (1) is of importance only as it relates to propagation

delay for very high data rates, (2) and (3) are worthy of note.

The standard data rate in 1987 is still 1200 baud.  There are

56kbps modems being beta tested now, but that is still only 6% of

1 Mbps.  Ownership by a single organization is also something

that is unusual in the amateur radio network.  Item (3) tends to

lead to either a homogeneous set of network hardware, a common

set of goals, or at least a common forum for discussing those

items.  In the amateur world, users and implementors in northern

California, Southern CA, and Arizona don't get together very

often.  That's another advantage of (1), in Los Angeles, one node

can cover a area with a diameter of 200 miles.

Many of the studies done on LAN performance make assumptions that

are not valid in the amateur environment.  One study, for

example, from [2] page 289 assumed:

o   All packets are of constant length

o   There are no errors, except those caused by collisions.

o   There is no capture effect.

o   Each station can sense the transmissions of all other

stations.

None of these assumptions hold for our current environment.

Another difference between our network and more commonly modeled

networks is in the large number of autonomous stations we have on

the network, and the large number of different traffic patterns

running simultaneously.  During the three days that data was

gathered for this paper, 371 transmitters were on the air at some

time in southern California.  The peak number of active

transmitters on a single 1200 baud frequency in a single five

minute interval was 42.  Most modeled networks have higher baud

rates and/or low data throughput, and assume traffic is moving

between a large number of outlying stations and a central

station.

Again, while there is much value in reading and modeling, we

should make the attempt to measure what we have; both to feed the

result back into the models, and to establish a base against

which future modifications can be judged.

2.  The Current State of Affairs.

There appears to be only one kind of monitoring being done in

amateur packet radio today.  The two most common BBS systems, by

W0RLI and by WA7MBL, both produce a log of BBS activities.  An

analysis program produces a report for the BBS operator of the

number of connects from users and the number of messages

forwarded, among other items.  While this gives a BBS operator

some idea of his local usage patterns, it does little to

described total network activity, or even the throughput the BBS

experiences.

For global network performance we are left with anecdotal

evidence, e.g., "01 really stinks tonight" (translation:

performance is less than expected), and "I had no problem with 01

today" (translation: I'm retired and was on at 10:00am).

For local user performance, we get "I can talk to Utah all night

long", and "I haven't been able to connect up north all week".

Obviously, we need something better.

3.  It's not easy

There are two ways of looking at network performance, one is from

the network's point of view, the other is from the user's point

of view.  In the first case we are interested in how the channel

is performing, in the simplest view, how many bytes of data it is

carrying.  Is the network carrying a large number of user bytes,

or is most of the capacity going to overhead or retries?  Are we

losing data to collisions, or to bad RF paths?

In the second case, the user's point of view, the questions are

more toward what level of service an individual user is getting

from the network.  Is the response time from distant locations

adequate?  Do many connections time-out?  Are some destinations

unreachable due to congestions or path failures?

There are several ways of acquiring performance data.  One is to

have each user station collect it.  As updating 40,000 user's is

a non-trivial exercise, we've chosen another route.  A specialized

monitor station sits at a central place and looks at all the

activity on the channel.  Unfortunately, it isn't easy to answer

any of the questions from a third party monitor station.  Some

of the problems are discussed below.

3.1   The problem is, it's Radio.

In most wire based, broadcast-type LANs, a monitor program can

make the assumption that if it heard a packet, everyone else in

the LAN heard the packet.  More importantly, if it didn't hear a

packet, no one else did either.  Even if the LAN is relaying data

between two other LANs, it is at least certain that for data

originated on the LAN or destined for the LAN, the monitor has a

high probability of having the seen the same data as the other

stations on the LAN.  In the amateur packet network, due to

hidden terminals, the FM capture effect, and propagation, all

stations do not hear the same packets.

If the monitor station heard all packets, it could easily follow

the state of all connections on the LAN.  For connection oriented

protocols like AX.25 and TCP, and providing the monitor has been

up as long as the other stations on the LAN, the monitor can tell

how long a connection has been in place based on the circuit

start and end protocols.  In the amateur radio case, the monitor

station can not be certain that it heard all packets.  It may

miss a circuit startup or end.  It must instead be prepared to

infer that a connection exists because it sees data flowing, or

that a circuit has closed because it has seen no data for an

interval of time.  This will add uncertainty to data gathered in

an RF environment but it does not invalidate the entire effort.

Although collisions can be directly detected on a wire LAN, they

can not be as easily detected on radio.  Due to the capture

effect, a stronger FM station will completely override a weaker

station such that stronger packet is received without error,

even though two packets were being transmitted at the same time.

A collision may be inferred if the received packet is seen again.

Some tasks then become exercises in gather as much information as

possible, and then making an educated guess.  Still, this is

better than no data at all.

3.2 Users are Easy to Replace.

It is somewhat easier to gather user oriented data, e.g., does a

path to station X exist at this time, or what is the round-trip

delay for packets between Los Angeles and Salt Lake City.  The

monitor station can actually be a user and directly measure these

values.

While data can be gathered about the performance of the channel at a

specific time in this way, this alone will not supply information

about the global network status at the time the measurement was

taken.  To be able to draw a meaningful conclusion from the data,

aside from variable X was equal to Y at time T, other information

is needed, such as the number transmitters on the air, and the

number of other packets on the channel.  In sort, both types of

monitoring must be performed, direct measurement of user

performance and global network measurement.

4.  Monitoring Software

The software currently under development by the authors addresses

the problem of global network monitoring.  Other types of

monitoring will be added in the future.

In this early version of the software, we are attempting to

determine what sorts of questions can be answered by a program

which listens to a channel and takes note of the packets it hears.

Some questions, such has how many total bytes are being

received at the monitor site, how many transmitters are seen, how

many beacons are heard, are easy to answer.

A much more difficult question is "How many times does the

average forwarding BBS send a 20k file before it goes all the way

without timing out?"  The type of information we're collecting,

and the type of questions that can be answered, are discussed

below.

4.1  Questions to Answer

There are two basic questions which are reasonably easy to answer.

One is "What is the efficiency of the channel", the other is "How

many users does the channel support".

We have chosen to define efficiency as the ratio of the number

of unique bytes of user data on the channel verses the total

number of bytes on the channel.  "Unique data bytes" is our term

for actual user data not including frame overhead, retransmitted

copies, or digipeated copies.  For example, if the string "hello"

is entered, digipeated once, not acked, retransmitted,

redigipeated, and acked, the total number of bytes on the channel

would be 168, the number of unique data bytes is 5, an efficiency

of 2.9%.  If 256 user bytes are sent and directly acked, the

efficiency is 88%.

To keep statistics on each user of the channel, we store pairs of

Source and Destination calls from the frame header.  The pair is

called a circuit.  A normal two-way connection would consist of

two circuits.  If NK6K and WB6YMH were connected, one circuit

would be (TO:NK6K,FROM:WB6YMH), the other circuit would be

(TO:WB6YMH,FROM:NK6K).  Statistics for each circuit are maintained

separately.

In addition to two basic questions, we wanted to be able to determine

the number of digipeaters the circuit used, what the average size

of a data frame was, the number of RNR (input blocked) frames

transmitted, and similar questions.  Since this required looking

into the control fields of the frame, the standard TNC interface

was unsuitable.

4.2  KISS

We chose the KISS TNC interface to give us access to all fields

of the frame.  KISS sends the entire frame, minus the checksum, to

the terminal port using an async framing format.  The KISS

interface has been implemented on the TAPR TNC 1,  the TAPR TNC 2

and clones, and on the AEA PK-232.  The KISS software for the TNC

2 is included with the KA9Q TCP/IP package.

There are no modifications required to the KISS code for use in

this application.

4.3  Software Design

The current implementation of the monitor package consists of

three programs.

o  STATS.EXE - This program monitors the received frames and

accumulates data, periodically dumping the data into a log file.

STATS also displays the addresses, data, control fields, and a

"retry" flag in real-time as frame are received.  NET/ROM and

TCP/IP control fields are also displayed.

o  REPORT.EXE - This program massages selected data from the log

file into a form suitable for passing to a plotting program.  The

plotting program is not included.

o  AVERAGE.EXE - This program massages the output of REPORT,

combining and averages the records into larger intervals of time.

This can result in clearer plots.

4.3.1  STATS.EXE

STATS collects data over a five minute interval, storing it into

several different tables.  These tables are then written into the

log file at the end of each interval, along with a time stamp

record.  The tables are summarized below.

Digipeater Data.

The total number of packets and bytes heard from a digipeater is

stored, along with the call of the digipeater.

Frequency Data.

Totals on bytes and packets heard on the channel without regard

to source are maintained.  Packet are also counted by length into

five buckets: 32, 64, 128, 256, and greater than 256 bytes.  The

total number of ticks of the 18.2 Hz clock when the data carrier

detect (DCD) line was high are recorded, as are the number of

ticks when DCD was low.

Circuit Data.

Several items are stored for each circuit, or TO:/FROM: pair.

This includes the number of digipeaters used, the Protocol ID

Byte (PID) of the last I frame received in the interval, the

total number of packets and bytes received, the number of unique

packets and bytes received, and the number of packets and bytes

ignoring those heard from multiple digipeaters.  Also included is

the number of unique frames heard of each frame type (sabm, ua,

etc.), the number of frames with POLL, and the number of frames

with FINAL.  The number of I frames heard is also counted into

five buckets based on the data size: 32, 64, 128, 256, and

greater than 256 bytes.

As an indication of the difficulty of accurately determining the

status of a frame, the algorithm used to determine uniqueness is

described below.

Uniqueness

Depending on the packet type one of three different algorithms

are used to test for uniqueness.

I frames are judged to be unique if the N(s) variable matches the

expected V(s), or if the locally computed checksum of the

information portion of the current frame does not match the

checksum of the last frame received with the same N(s).  Note

that the checksum is only used to resolve the ambiguity resulting

from lost frames.  An algorithm based solely on checksums would

be confused easily by data streams containing identical

consecutive lines.  For example consider the transmission of text

files containing multiple blank lines separating pages.  In such

cases several consecutive packets would contain identical

information, a single carriage return, but still be unique.

S and U type frames are judged to be unique if the control field

of the current frame is different than the control field the last

S or U frame received.  Note that this does not detect retries of

frames such as multiple SABMs sent because the target station is

not responding.

UI frames are judged to be unique frames if the checksum of the

information field of the current frame is different than the

checksum of the last UI frame which was received.

Digipeated frame filtering logic

The various "non-digipeated" counters in the software are

designed to show the number of times a particular frame appears

on the channel without regard to multiple retransmissions by

digipeaters.  The "non-digipeated" counters are advanced once and

only once regardless of how many digipeater hops are observable

by the monitoring station.  This data is used to determine the

number of retries of a packet without confusing a retry for a

digipeat.

The software maintains bit maps of observed hops for its use in

filtering out digipeated frames. A separate bit map of observed

hops is maintained for UI, S and U frames types as well one bit

map for each outstanding I frame. There are 9 bits in each map

which correspond to the originating station plus up to 8

digipeaters.

A frame is considered to be "non-digipeated" when it is either

heard for the first time or it is heard from a hop from which it

had been previously heard.  The first condition is met when a

frame is first transmitted, the second condition is met when a

frame is retransmitted successive times. If neither case is met

the frame is a digipeated frame and is not used to increment the

"non-digipeated" counters.

The digipeat bit map is cleared when either the uniqueness

subroutine determines the frame is unique or when the digipeat

filter subroutine determines that the frame is a retransmission.

4.3.2  REPORT.EXE

REPORT produces several output formats.  The RAW format displays

each field in each record.  This is useful if a particular

interval is being examined in detail, or when debugging STATS.

Several other formats are used to produce data for plotting.  One

report totals all circuit data for an interval.  Examples of this

output are provided later.

4.4  Hardware

As discussed above in the section on KISS, TNC 1 and TNC 2

clones, and the AEA PK-232 can be used with this software.  If the

DCD ON and OFF times are desired, a jumper must be added.

DCD Jumpering

Since most of the current TNC designs use the DCD signal on the RS-232

interface as a connect status indicator it is necessary to modify

the TNC hardware slightly to provide a true modem DCD on the RS-232

interface.  The modification for the TNC 2 and clones is very

simple, consisting of a single jumper wire.  The jumper goes

between pin 2 of the modem disconnect header (DCD output from the

modem) and the pin of JMP1 which is NOT connected to +5 volts

(input to the DCD driver).  On the MFJ-1270B artwork the correct

pin of JMP1 is the one closest to the front panel.  The authors

have not researched modifications to other TNC designs, but it is

expected the modifications will be similar.  It is NOT necessary

to perform the DCD modification to run the monitoring software,

it is only necessary if the statistics of DCD activity are

desired.

Most terminal software used on packet will be unaffected by this

modification, however most BBS software will require the jumper to

be removed for normal operation.

The software was developed on an IBM PC/AT using Microsoft C 4.0.

It should be easily transportable to other systems provided a

suitable serial port interface is available.

A hard disk is highly recommended.  Twenty-four hours of data for

145.01 MHz as monitored in southern California produced 500k

bytes of log file data.  This may be reduced, of course, by

increasing the interval time.

5.  Examples

We used the STATS program to acquire performance data on all of

the active packet channels in southern California.  The

monitoring site used for 145.01 MHz was at 700 feet on Palos

Verdes.  On this frequency, the site can "see" 8 NET/ROM nodes.

During the 24 hours during which 145.01 was monitored, from 00:00

to 23:59 local time on a Thursday, 105 total transmitters were

seen.

The data shown in the sample graphs is based on the five minute

interval data from STATS, which was then processed by REPORT.

The output of REPORT was then averaged by AVERAGE into 15 minute

samples.  Each point plotted represents the average of three five

minute intervals.

Figure 1 shows the number of user circuits seen in an interval.

A "user circuit" is a subset of the total circuits; beacons,

repeater ids, and other circuits consisting of a small number of

UI frames have been removed.  The peak of data just after midnight

is caused by forwarded BBS traffic, large broadcast messages such

as newsletters are restricted to being forwarded between 00:00

and 8:00 by local custom.

Figure 2 shows the total bytes per minute.  Further analysis of

the data would show the distribution of bytes in the peaks; how

much is destined for local users, and how much is going

"overhead", passing through the backbone to other NET/ROM

locations.  The major NET/ROM path through to Arizona is still on

145.01, this should change before the summer is out.  It will be

interesting to see what effect moving the backbone will have on

this graph.

Figure 3 shows the efficiency of the channel, computed as

discussed above.

Figure 4 is a plot of efficiency vs. the number of user circuits

on the channel.  The distribution on a plot of efficiency vs.

total packets is similar to this one.  The occurrence of low

efficiency over the entire range of users (and number of packets)

shows that there are causes of low efficiency other than

congestion.  One interpretation would be that more data is lost

due to poor RF paths than to collisions.  Another would be that

hidden terminals are causing problems.  Further analysis of the

data, coupled with a knowledge of the geography and stations

involved, might result in information that could be used to

improve the network.

6.  Other Uses / Future Goals

Once a basic set of data gathering tools and formats has been

define, the applications are boundless.  For example, STATS can

be used to make improvements to the current 14.109 HF forwarding

scheme.  For example, data gathered during the day on Friday,

July 29, shows that of the two top stations in terms of the total

bytes transmitted, shows that one had 30% better efficiency than

the other.  If monitoring was continued, and the trend continued

over time, it may mean that the less efficient station is trying

to reach stations beyond its range, or that there are local

receiver problems.  It also means that the monitor station was

hearing more data frames from the transmitting station that the

target station was, perhaps the mail between those stations

should be re-routed.

STATS can be used to check propagation between the monitor

station and other stations.  Figure 5 shows then number of bytes

received on 14.109 MHz in a 24 hour period. It can also be used to

infer propagation between other station.  For example, if you

hear a station in Indiana sending packets to Seattle and the

efficiency is high, then a path must exist between those two

points, even if you do not hear packets from Seattle at the

monitor site.

The "unique" subroutine can be used filter retries out of a

monitored connection as the data of the connection is displayed.

AEA offers a similar feature on some of its TNCs.  STATS will be

updated in the future to allow the capture of filtered text from

each circuit into files for later review.  This can serve several

purposes, as a diagnostic aid, a periodic check for intruders on

the amateur network as required by the FCC, or to satisfy the

standard urge to "read the mail".

This type of data collection could also assist in message traffic

analysis, e.g., how many bytes are in the average connection?

Are most of the BBS messages forwarded on a channel destined for

users in the local area or are they just passing through?

Currently, STATS monitors at the link-layer level.  Higher layer

protocols such as TCP/IP and NET/ROM add additional complications

to traffic analysis, primarily in determining the actual

origination and destination point.  Work remains to be done in

this area.

7.  Conclusion

There is much good to be gained from gathering and analyzing

performance data.  It can tell us where we are and suggest where

we might go.  It will also help determine if we like where we've

gone once we get there.  The work discussed here is a start

toward developing tools to aid in this task.  Others are invited

to participate.

8.  Availability

The software described in this paper is available in source form

from the WB6YMH-2 BBS on 145.36 in southern California.  This BBS

is also available by phone for those not in the local area at

(213) 541-2503.  Updates will periodically be sent to the HAMNET

BBS on Compuserve.

9. Acknowledgments

Thanks to Craig Robins, WB6FVC, for his help in the preparation

of this paper.  Thanks also to those who have implemented the

KISS code for the TNC 1 and TNC 2, and the folks at AEA.

10. References.

[1]  Karn, P., KA9Q, "Proposed Changes to AX.25 Level 2", informal

paper circulated on various mail systems and reprinted in the

July/August 1987 NEPRA PacketEar, the newsletter of the New

England Packet Radio Association.

[2] Tanenbaum, A., "Computer Networks", Englewood Cliffs, NJ:

Prentice Hall, 1981.

#######