A Front End Filter for the GRAPES 56 kbps Modem

[Downloaded from hydra.carleton.ca /pub/hamradio/packet/modems 2-25-95 by WÝTN]

The GRAPES 56 kbps modem has a very broad front end response, having only lowpass filtering.  The following circuit is a 28-30 MHz bandpass filter (designed by Dennis Rosenauer, VE7BPE) which can be inserted between the modem input and the output of a transverter or receive converter.  It will help eliminate problems with out-of-band signals and reduce DCD falsing.

C3             C5

-------+---||---+-----+---||---+-------

|      |        |     |        |      |

C1 _|_  L _|_   C4 _|_  L_|_    L _|_    _|_ C6

50 ohm in       ---    | |      ---   | |      | |    ---      50 ohm out

(from xvtr)       |     |_|       |    |_|      |_|     |       (to modem)

>----------|     _|_      _|_   _|_      _|_     |--------->

>---   C2 _|_   /  /     /  /  /  /     /  /    _|_ C7  --->

_|_     ---                                   ---    _|_

/  /      |                                     |    /  /

_|_                                   _|_

/  /                                  /  /

Parts list:

C1: 75 pF

C2: 300 pF

C3: 3.9 pF

C4: 56 pF

C5: 4.7 pF

C6: 68 pF

C7: 470 pF

L:  0.47 uH slug-tuned coil (Toko BTKXNS-1048Z, same as used in

the GRAPES modem. Q = 70)

Capacitors are standard 5% values, and should be good quality (NPO ceramic

or polypropylene).

For best results, the filter should be tuned up using a spectrum analyzer

with tracking generator or a network analyzer, but tuning for maximum

output at the frequency of interest may be adequate.  When properly tuned,

the filter has a flat passband 2.5 - 3 MHz wide, about 4.5 dB insertion

loss, and is about 40 dB down at 25 and 35 MHz.

Rendered into ASCII by VE3JF

Ottawa ARC Packet Working Group