Receiving Bits Using a PIC

int bitin(){  					//function to read a bit

static int oldstate;  				//oldstate retained between runs of this function

int k;

for (k=0;k<121;k++){ 		//this loop allows 838 us to go by.

// If no state change, bit is 1

if (input(rcvPin) != oldstate){   	//if state has changed

oldstate = input(rcvPin);  	//update oldstate

delay_us(430); 		// move to halfway thru the next bit

return 0;  			//return 0 if state changed

}

}

return 1;  					//return 1 if state did not change

}

Looking for the First Flag

int cbyte = 0;       				//initialize

while (cbyte != 0x7e){  			//find the first flag

shift_right(&cbyte,1,bitin());  	//add a new bit to the left of cbyte,

//discard right bit

}

output_high(LED);  				//turn on the DCD light

Get All the Other Flags

int i;

int bte[40];

While (cbyte = = 0x7e){  			//find the other flags

for(i=0;i<8;i++){			//repeat this 8 times

shift_right(&cbyte,1,bitin()); 	//add a new bit to the left of cbyte,

//discard right bit

}

}							//end of while - now at end of all the flags

bte[0] = cbyte; 				//you've now got the first address byte

//put received data in bte[ ] array

Get the Rest of the Transmitted Data

int test, newbit, numbyte, ones;

test =0;

numbyte = 1;						 //we already collected 1 byte

while (test != 1){ 					 //do this until the flag at the end of the packet

for(i=0;i<8;i++){				 //repeat 8 times to collect 8 bits

newbit = bitin();			 	 //get a bit

if (newbit = = 1) (ones++);  		 //increment the ones counter

else (ones = 0);			 //if bit is a zero, reset the ones counter

if (ones = = 5) {  				//removes bit stuffing

test = bitin();  			//get the next bit but don’t add it to cbyte

ones = 0; 				//reset the ones counter

}

shift_right(&cbyte,1,newbit);               	//append the new bit to cbyte

}							//end of for

if (test = = 0){

bte[numbyte] = cbyte;			//add cbyte to the array

numbyte++;				//increment the number of received bytes

}

}

Computing the FCS

if (fcscheck()) (printout());

short fcscheck(){  					//checks the fcs

int i,k,bt,inbyte, fcslo, fcshi;

fcslo=fcshi=0xFF;					//intialize FCS values

for (i = 0;i<(numbyte-2);i++){			//calculate the FCS for all except the last 2bytes

inbyte = bte[i];

for(k=0;k<8;k++){				//perform this procedure for each bit in the byte

bt = inbyte & 0x01;

#asm					//embedded assembly language route to do a 16  				BCF    03,0				//bit rotate

RRF    fcshi,F

RRF    fcslo,F

#endasm

if (((status & 0x01)^(bt)) ==0x01){

fcshi = fcshi^0x84;

fcslo = fcslo^0x08;

}						//end of if

rotate_right(&inbyte,1);         		//set up to do the next bit

}							//end of for

}

fcslo = fcslo^0xff;

fcshi = fcshi^0xff;

if ((bte[numbyte-1] == fcshi) && (bte[numbyte-2] == fcslo)) {

return 1;

}

else return 0;					//if the computed values equal the last 2 data bytes

}								//return a 1, otherwise return a 0