Version 3/4/97 by Paul Beckmann -- notes to the group are in {..} Please comment to me directly so I can include the revisions in my master copy (wa0rse@amsat.org). Also, a number of tbd's were in the 1/4/97 BOM from W3IWI. You'll find them below. This document currently reflects a  "full population" of the board. The intention is to make  a number of sub-documents based on this document by cutting out what is not  necessary for a given version. PJB 3/4/97.

Extensive revisions 3/22/97 by W3IWI to reflect my ideas on the build and to convert document to Microsoft  Word.

Tucson Amateur Packet Radio

8987-309 E. Tanque Verde Rd #337

Tucson, Arizona 85749-9399

Office: (817) 383-0000 Fax: (817) 566-2544

Non-Profit Research and Development Corporation

TAC-2 Assembly Manual

This document was written February 1997. Rev 0: Month dd, 199y.

Copyright 1997 Tucson Amateur Packet Radio Corp.

The information contained in this document has been carefully checked and is believed to be entirely reliable. However, no responsibility is assumed for inaccuracies. Tucson Amateur Packet Radio Corporation (TAPR) reserves the right to make changes in any products to improve reliability, function or design without obligation to purchasers of previous equipment. TAPR does not assume any liability arising out of the application or use of any product or circuit described herein; neither does it convey license under its patent rights or the rights of others. Reproduction or translation of any part of this work beyond that permitted by sections 107 or 108 of the 1976 United States Copyright Act (or its legal successor) without the express written permission of Tucson Amateur Packet Radio Corporation is unlawful except as noted below. Requests for permission to copy or for further information should be addressed to Tucson Amateur Packet Radio Corporation. Except as noted above, permission is hereby granted to any non-profit group or individual to reproduce any portion of this document provided that: the reproduction is not sold for profit; the intent of the reproduction is to further disseminate information on Amateur Packet Radio; the reproduction is not used for advertising or otherwise promoting any specific commercial product; full credit is given to Tucson Amateur Packet Radio Corporation (including address) as the original source of information; and Tucson

Amateur Packet Radio Corporation is notified in writing of the reproduction.

Table of Contents

- INTRODUCTION

- PARTS LIST

- CONSTRUCTION

- GENERAL INSTALLATION GUIDE

- SPECIFIC UNIT INSTALLATION

- Garmin GPS-20

- Motorola Oncore Basic

- Motorola Oncore VP

- Trimble SK8

- OPERATION

- THEORY OF OPERATION

- SCHEMATIC DIAGRAM

- BOARD LAYOUT

- INTRODUCTION

The “Totally Accurate Clock” TAC-2 printed circuit board (PCB) is intended to serve several purposes:

- It provides a “universal” electrical and mechanical interface for a number of common OEM board-level GPS receivers including specifically

- Garmin GPS-20

- Motorola Oncore

- Trimble SK8

- It provides interfaces for the 1 pulse-per-second (1PPS) signal generated by these receivers:

- Low-impedance, fast rise-time 1PPS signals for “laboratory” applications.

- RS232 level 1PPS signals for computer applications.

- Specialized 1PPS interfaces for an add-on PCB (soon to be announced by TAPR) that will stabilize a low-cost crystal oscillator to an accuracy ~ 1 part-per-billion

- It provides several different power supply options to make your use of your GPS receiver easier. The power interface is similar to (and compatible with) that used on computer disk drives

- A low-cost, lossy (7805-type)  regulator.

- A high-efficiency switching power supply.

- Direct 5 volt power.

- It has provision for an Uninteruptable Power Supply (UPS) to buffer the GPS receiver through brief power outages.

- It provides Battery Backup so the GPS receiver can wake up “smart”

- It can provide isolated power for an amplified GPS antenna

Since the TAC-2 has so much flexibility, there are several options you will have to exercise during the construction. The “base” implementation will satisfy the needs of many amateurs, with support for the Garmin GPS-20 and Motorola ONCORE Basic receivers. A few jumper changes enable support for the Motorola ONCORE VP/UT receiver. Some added parts and some more jumper changes are needed if you plan to use a Trimble SK-8. Overall, construction and check-out of the TAC-2 will probably take you only an evening or two.

Warm up your soldering iron, take your time, and enjoy!

Note: Find and read your TNC installation before continuing, to decide on

specific installations before building the kit.

2. PARTS LIST

Sort the parts into egg cartons, muffin tins, or other containers as you

inventory them. This will aid you in building.

Check off the part in the [_] space provided as you locate and verify it

against this list.

Base Configuration:

Resistors

1/4 watt, 5%:

[_] 2@		510 ohm (green-brown-brown-gold)

[_] 1@		1k ohm (brown-black-red-gold)

[_] 1@		1 Megohm (brown-black-green-gold)

[_]  tbd 	(?R3-R8)

8-pin Single Inline Package (SIP) Resistor Packs:

[_] 2@		4 x 47 ohm (470)

[_] 1@		4 x 2.2 kohm (222)

[_] 2@		4 x 10 kohm (103)

[_] 1@		100 ohm potentiometer

Capacitors: Capacitors may be marked in various ways. The typical markings are given but may vary. Find all that match the typical markings given and the remaining ones, if any, should become apparent by elimination.

[_] 1@		470 pF (471) (Mylar or Monolithic)

[_] 12@	10 nF(103) (Mylar or Monolithic)

[_] 1@		100nF (104) (Mylar or Monolithic)

[_] 8@		1 or 2.2 uf  Electrolytic or Tantalum capacitors

Semiconductors: Integrated Circuits come from various manufacturers and may have differing prefixes and/or suffixes. For example, if the part is listed as a 74LS00, it may be marked SN74LS00N or MC74LS00P or DM74LS00N or F74LS00P or some other variation. The key is that the sequence 74LS00 appears in the part number. A four-digit number, such as 8834, indicates the year and week of manufacture and should not be confused with the part number.

NOTE: Do not handle the ICs at this time! Carefully remove the black foam carrier with ICs from the bag and verify the ICs against this list. Then return the foam with the ICs to the bag. Do not touch the ICs!

[_] 1@		74HC132 (14 pin DIP)

[_] 1@		MAX232, ISL232CPE, TC232CPE (16 pin DIP)

[_] 2@		74AC04 (14 pin DIP)

[_] 1@		LM7805 3-terminal Regulator

[_] 7@		1N4148 Silicon Signal Diode

[_] 2@		1N4001 Silicon Power Diode

[_] 1@		LED

Radio-frequency Choke:

[_] 1@		6.8 uh choke

Sockets and Connectors:

[_] 3@		14-pin DIP IC Sockets

[_] 1@		16-pin DIP IC Socket

[_] 1@		4 pin polarized MOLEX header

[_] 2@		2x4 headers

[_] 3@		2x5 headers

[_] 1@		1x5 header

[_] 3@		2x5 IDC jacks with 10-wire ribbon cable

[_] 2@		DB9F IDC connector

[_] 1@		1x4 connector jack with pins

[_] 1@		1x5 connector jack with pins

Mechanical Parts:

[_] 4@		3/8”  4-40 threaded standoffs for GPS engine

[_] 4@		3/4”  4-40 threaded standoffs for TAC-2 PCB

[_] 20@	1/4”  4-40 screws

[_] 20@	#4 lockwashers

[_] 2@		4-40 nuts

Additional Parts for High-efficiency Switching Power Supply:

[_] 1@		Power Trends PT5101 or 78SR Switcher

[_] 1@		220 uF Electrolytic Capacitor

[_] 1@		6.2V Zener Diode

[_] 1@		6.8 uh choke

[_] 1@		10 nF Capacitor (103)

Additional parts required if you need to supply a BBRAM keep-alive battery (not required for Garmin GPS-20 supplied by TAPR):

[_] 1@		1x2 header

[_] 1@		180 ohm resistor (brown-gray-brown-gold)

[_] 1@		1N4148 Silicon Signal Diode

[_] 1@		5 Farad “Gold” Capacitor or 3.6v NiCd Battery

Additional parts required for Motorola ONCORE Basic receiver

[_] 1@		2x5 header

[_] 2@		2x5 IDC jacks

Additional parts for both Motorola ONCORE VP/UT and Trimble SK-8 receivers:

[_] 1@		1x3 header

[_] 1@		2x5 header jack

[_] 1@		bag with ten 2-pin jumpers

Additional parts required for Trimble SK-8 receiver:

[_] 1@		7555 Integrated Circuit (8 pin DIP)

[_] 1@		2N3906 PNP transistor

[_] 1@		8-pin DIP Socket

[_] 1@		1 Megohm Resistor (brown-black-green-gold)

[_] 1@		10 nF Capacitor (103)

[_] 1@		100 nF Capacitor (104)

[_] 1@		2x4 header

[_] 1@		2x5 header

Additional Parts if you want to use the optional Antenna Bias supply

[_] 1@		LM78L05 3-terminal regulator

[_] 1@		1N4001 Power Diode

[_] 2@		10 nF Capacitor (103)

[_] 1@		1 uF or 2.2 uf Electrolytic or Tantalum

[_] 1@		500 ohm potentiometer

[_] 1@		510 ohm resistor (green-brown-brown-gold)

[_] 1@		1x2 header

[_] 1@		Zener Diode (optional, voltage as required)

You are now ready to begin construction of the TAC-2. Use a temperature-controlled, fine-tipped soldering iron and good quality 60/40 or 63/37 rosin-core solder for construction. Follow standard construction practices when building the unit.

In addition to the soldering iron and solder, you will need small flush or semi-flush cutting pliers and small-tipped long nosed pliers. A magnifying glass may prove helpful to identify the values of the small components.

Pay careful attention to the directions that follow. Keep the tip of your soldering iron bright and clean, wiping it frequently on a wet rag or sponge. Make solder joints carefully, but swiftly. Prolonged heat on a PC board pad can be as disastrous as too much heat, and ruining this PC board can be expensive! Two to three seconds should be enough time to apply heat to any joint.

IC Sockets and connectors:

NOTE: If any socket pins are bent, carefully straighten them with a pair of long-nose pliers before assembly. Some types of IC sockets have crimps in the pins to hold them in place when automatic wave-soldering is performed. These sockets may be tricky to install if you are not familiar with them. If your kit contains these sockets, you may want to straighten the pins before attempting to insert them into the PC board.

When installing IC sockets, double check to ensure that the socket is seated properly against the board with the notch matching the silkscreen. Pin 1 (nearest the socket notch) has a square solder pad. Be sure that all IC socket pins are showing on the solder side of the board. Next, tack-solder two diagonally opposite corners first (such as pins 1 and 8 on a 14-pin socket).

Then solder the remaining pins of that socket before proceeding to the next one. If you find a socket is difficult to install, remove it and double-check for a bent pin.

NOTE: Take care now to avoid solder bridges!

Install the following IC sockets.

[_] U1 	14-pin

[_] U2 	16-pin

[_] U3 	14-pin

[_] U4 	14-pin

Now check your work. All leads should be soldered. There should be no solder

bridges (a blob of solder that shorts two adjacent soldered connections) or

cold (gray and/or grainy looking) solder connections. You should have no IC

sockets remaining.

[_] OK so far.

Header connectors are supplied for I/O connections. First we install the headers for the RS232 I/O. Install them using the same procedures you used for the IC sockets. Note that the PCB silkscreen has a square to denote pin #1.

[_] P1 	2x5 pin header

[_] P2 	2x5 pin header

[_] P3 2x5 pin header

If you will be using a Motorola ONCORE BASIC GPS receiver (this is Motorola’s original receiver, also called the PVT-6 with a PCB 100mm x 70mm in size), then also install:

[_] P4 	2x5 pin header

{Note: The headers at P5 and P6 are intended for future expansion.

Don’t worry about them at this time.}

The following connector is polarized. Install it with the plastic polarizing tab away from the edge of the board and towards the diode D8, matching the silkscreen artwork. This connector matches the 4-pin connector used on many disk drives:

[_] P7 	4 pin polarized MOLEX connector

[_] OK so far.

Resistors & Potentiometer

Resistors lie flat on the PC board and have a lead spacing of 0.400". You may wish to use a lead former to preform the resistor leads for neatest appearance.

Install the following 5% resistors:

[_] R1 	1 Megohm (brown-black-green-gold)

[_] R2 	510 ohm (green-brown-brown-gold)

{W3IWI note – the following are needed only for the lossy 7805

regulator and probably should be in the power supply section}

[_] R10 510 ohm (green-brown-brown-gold)

[_] R11 ceramic potentiometer. The ceramic body of this part

should be perpendicular to the circuit board and the gray

knob should face ???

WARNING! Be careful when clipping leads, as they have a tendency to fly towards your eyes! Take appropriate precautions (grasp leads and wear eye protection).

[_] Solder and clip the leads (9 total)

SIP (Single Inline Package) resistors used in the TAC-2 are symmetric and not polarized.  It is recommended that you match the dot or Pin 1 on the resistor with the pad marked with a square on the silkscreen. All RPx Resistor Packs are 8-lead parts containing 4 identical resistors. The resistors should end up flush with the circuit board. It is easiest to solder the end pins first, then inspect the placement, and the solder the other 6 pins (just like you did by soldering the corner pins on the IC sockets).

[_] RP1		4x2.2k

[_] RP2		4x10k

[_] RP3		4x10k

[_] Solder the leads (24 total)

[_] RP4		4x47

[_] RP5		4x47

[_] Solder the leads (16 total)

[_] OK so far.

Diodes: Diodes are polarized, with the cathode end being banded. Observe polarity when installing the following diodes. The diode band must match the band shown on the PCB’s silk screen. Leads are at a 0.3" spacing. Diodes D1 through D7 are located between RP1 and RP2.

[_] D1 1N4148      	{ the cathode band on these 4

[_] D3 1N4148	  diodes all point towards RP2 }

[_] D7 1N4148

[_] D5 1N4148

[_] Solder and clip the leads (8 total)

[_] D4 1N4148	{ the cathode band on these 3

[_] D2	1N4148	  diodes all point towards RP1 }

[_] D6	1N4148

[_] Solder and clip the leads (6 total)

[_] D8	1N4001

[_] D9	1N4001

[_] Solder and clip the leads (4 total). The 1N4001 diodes

have heavy leads. Make sure that they get hot

enough for solder to flow properly!

Capacitors

NOTE: All capacitors should be mounted as nearly flush to the board surface as practical without stressing the leads. The first capacitors to install are located on the upper right-hand quadrant  of the PCB:

[_] C1		100nf	(104)		R1*C1 = 100 msec "squelch"

[_] C2		470pf	(471)		RP3*C2 = 5 usec pulse width

[_] CB1	10nf	(103)

[_] CB6	10nf	(103)

[_] CB7	10nf	(103)

[_] CB15	10nf	(103)

[_] Solder and clip the leads (12 total)

Now work in the top left-hand quadrant:

[_] CB11	10nf	(103)

[_] CB12	10nf	(103)

[_] CB13	10nf	(103)

[_] CB2	10nf	(103)

[_] CB16	10nf	(103)

[_] Solder and clip the leads (10 total)

In the bottom half of the board:

[_] CB10	10nf	(103)

[_] CB9	10nf	(103)

[_] CB14	10nf	(103)

[_] CB8	10nf	(103)

[_] CB4	10nf	(103)

[_] CB3	10nf	(103)

[_] Solder and clip the leads (12 total)

The following capacitors are polarized. The positive (+) lead should go in

the square pad towards R5. Electrolytic capacitors (tubular metal cans with

the leads coming out of one end) have the negative lead marked with a bold

black band; tantalum capacitors (teardrop-shaped epoxy-covered blobs) have

the positive lead marked with hard-to-read little plus (+) signs.

[_] CT5		1 or 2.2 uf

[_] CT6		1 or 2.2 uf

[_] CT7		1 or 2.2 uf

[_] Solder and clip the leads (6 total)

The following capacitors are immediately adjacent to U2 (MAX232).

All the “+” signs should point away from U2 and toward U5/R9.

[_] CT8		1 or 2.2 uf

[_] CT1		1 or 2.2 uf

[_] CT2		1 or 2.2 uf

[_] CT3		1 or 2.2 uf

[_] CT4		1 or 2.2 uf

[_] Solder and clip the leads (10 total)

Now check your work. All leads should be soldered. There should be no solder

bridges or cold solder connections. You should have no resistors, diodes or

capacitors remaining.

[_] OK so far.

Remaining Components:

Power Supply: Let’s take a break while you make a decision. There are 3 ways to power the TAC, and you need to decide which you want to use. So pick one:

[_] “Cheap” power with a lossy 7805 regulator

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

W3IWI note: I ran out of steam here

My comments on Paul’s original text are in {..} braces

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

NOTE: Bend the leads of the 7805 voltage regulator so that it lies over the

silk-screen outline of the device.

NOTE: For Garmin GPS-20 operation, the 7805 voltage regulator requires a

heat sink.  Install the supplied heatsink before soldering the 7805 to the

PCB.  Once the 7805 is soldered to the PCB, ensure that the fins of the

heatsink do not touch any other components or a short may result!

[_] Install Heatsink (recommended for Garming GPS-20 installation) {IWI note:

won’t be necessary. The PCB has more than enuff foil to not need a heatsink}

[_] U6 7805

For high efficiency regulation under a wide variety of input voltages, the

following substitution may be made:

[_] U6 Power Trends PT5101 or 78SR Switcher

{I'm not sure how to write up U7 here.}

{IWI: U7 and associated parts are there only for a few special applications.

The notes should mention the existence, but not give a detailed build}

[_] Solder and clip the leads (3 total)

Transistor

Transistors have 3-leads. The orientation of the tab on a cylindrical case

or the flat side of a non-cylindrical cased unit is shown clearly on the

silkscreen {IWI: only need for the Trimble}

[_] Q1	PNP		Needed only for dual port rcvrs

[_] Solder and clip the leads (3 total)

Carefully check the board for solder splashes, solder bridges and cold

solder joints.

[_] OK so far.

RF Chokes

[_] RFC1	6.8uh

[_] RFC2	6.8uh	{IWI: New Rev.C artwork has a default jumper across

RFC2. Needed only for the Switcher or in case of bad

RFI problems}.

[_] Solder and clip the leads (4 total)

Headers, Jacks and Pads {IWI: I put the essentials earlier}

[_] J1		2x5 Jack		ONCORE VP/UT I/O

[_] Solder the leads (10 total)

[_] J2		2x5 Jack		TRIMBLE SK-8 I/O (only 8 pins used)

[_] Solder the leads (10 total)

{CFG #1/#2 are normally not used. They serve a purpose only if the user has

a non-standard RS232 port. JP2 is 2x4 UNLESS the user has SK-8 when it

needs to be 2x5.}

[_] JP1		2x4 Header		CFG Port #1

[_] Solder the leads (8 total)

[_] JP2		2x5 Header		CFG #2 (2x4 OK for single-port rcvrs)

[_] Solder the leads (10 total)

{JP3 is not needed for Garmin or ONCORE BASIC. User will install it as a

part of the “what receiver do you have? Section.}

[_] JP3		1x3 Header		TXD Polarity Selection

[_] Solder the leads (3 total)

{JP4 is only needed for SK8 or other bizzare rcvrs. Default Jumpers handle

all but the basket cases}

[_] JP4		2x4 Header		1PPS CFG

[_] Solder the leads (8 total)

{JP5 will probably never be used. No header needs to be installed}

[_] JP5		2x2 Header		ONCR BASIC: +5v or +12v POWER

[_] Solder the leads (4 total)

{Something needs to be in here about soldering the cable for the GPS-20

to the board}

[_] JP6		Solder Pads		GARMIN GPS-20

{Not sure what to do with these next two}

[_] JP7		Solder Pads		SPECIAL RCVR (use with JP6 pads)

[_] JP8		Solder Pads		Alternate +12 IN

[_] JP9		1x5 Header		UPS POWER

[_] Solder the leads (5 total)

[_] JP11	1x2 Header		EXT BAT

[_] JP12	1x2 Header		ANT BIAS

[_] JP13	1x2 Header		Ax Output

[_] JP14	1x2 Header		Bx Output

[_] Solder the leads (8 total)

Carefully check the board for solder splashes, solder bridges and cold

solder joints.

[_] OK so far.

Cable construction

The following cables need to be made up to connect the TAC-2 board to

associated equipment. Use connectors that mate to the following plugs to

make up the cables:

[_] P1	2x5 Header		RS232 Port #1

[_] P2	2x5 Header		RS232 Port #2

[_] P3	2x5 Header		1PPS OUTPUTS

[_] P4	2x5 Header		ONCORE BASIC I/O

[_] P5	2x5 Header		EXPANSION #1

[_] P6	2x5 Header		EXPANSION #2

[_] P7	4 pin MOLEX		POWER IN (Disk Drive Compatible)

*** Need to put a piece in here that has the individual test the voltages

first then install ICs.  Otherwise they'll blow their GPS engines!  -- Steve ***

Observing static precautions and polarity, install the following ICs in

their sockets.  Pay particular attention to aligning the notch of the IC

with the socket:

[_] U1 74HC132

[_] U2 MAX232, ISL232CPE, TC232CPE

[_] U3 74AC04

[_] U4 74AC04

[_] U5 7555

You are now ready to proceed with installation of the GPS engine with your

TAC-2.

GENERAL INSTALLATION GUIDE

SPECIFIC UNIT INSTALLATION

Garmin GPS-20

Motorola Oncore VP

Motorola Oncore Basic

Trimble SV6 {no -- SK8. SV6 is another special basket case}

OPERATION

THEORY OF OPERATION

SCHEMATIC DIAGRAM

BOARD LAYOUT

**** Should we include a TROUBLESHOOTING section?  Since the TAC is mostly digital, a logic probe can find most of the problems -- Steve ****